Phần 2. YÊU CẦU VỀ KỸ THUẬT

Chương V. Yêu cầu về kỹ thuật

Yêu cầu về kỹ thuật bao gồm các nội dung cơ bản như sau:

Mục 1. Yêu cầu về kỹ thuật

- Giới thiệu chung về dự án và gói thầu

-	Tên gói thầu: “Mua sắm vật tư quang phục vụ hoạt động nhỏ lẻ của các chi nhánh, trung tâm SCTV năm 2025 - đợt 2”.

-	Tên dự toán mua sắm: Vật tư, thiết bị, nhân công phục vụ hoạt động nhỏ lẻ của các chi nhánh, trung tâm SCTV năm 2025 - đợt 2.

-	Địa điểm thực hiện dự án: Do chủ đầu tư chỉ định (trong khu vực TP Hồ Chí Minh).

-	Hình thức hợp đồng: Hợp đồng trọn gói.

-	Thời gian thực hiện gói thầu: 60 ngày (Trong đó thời gian giao hàng: tối đa 30 ngày kể từ ngày kí hợp đồng).

- Mô tả hàng hoá

| STT | DANH MỤC HÀNG HÓA | ĐVT | SỐ LƯỢNG | Ghi chú |
| --- | --- | --- | --- | --- |
| 1 | Mô-đun quang XFP TX1270nm, 60km, truyền dẫn hai chiều, hỗ trợ giám sát đo đạc thông số điện tử (Module quang XFP TX1270nm, BIDI, 60km support DDM) | Cái | 2 | Không yêu cầu giấy phép bán hàng, giấy ủy quyền bán hàng của nhà sản xuất hoặc của đại lý phân phối hoặc giấy chứng nhận quan hệ đối tác hoặc giấy cam kết hỗ trợ kỹ thuật, bảo hành của nhà sản xuất hoặc của đại lý phân phối hoặc tài liệu khác có giá trị tương đương. |
| 2 | Mô-đun quang SFP+ 10G, TX1330nm, RX1270 , 60km, truyền dẫn hai chiều (Module quang SFP+ 10G, BIDI, 60km (TX1330nm, RX1270nm)) | Cái | 1 |  |
| 3 | Mô-đun SFP+ 10GE, 850nm, 300m hỗ trợ giám sát đo đạc thông số điện tử (SFP+ 10GE, 850nm, 300m support DDM) | Cái | 2 |  |
| 4 | Mô-đun XFP 10GE, 850nm, 300m hỗ trợ giám sát đo đạc thông số điện tử (XFP 10GE, 850nm 300m support DDM) | Cái | 2 |  |
| 5 | Hộp phối quang 12 cổng (Tối đa 1RU) (kèm dây nhảy, ống nung) (ODF 12 ports (max1RU) (kèm dây nhảy, ống nung)) | Cái | 2 |  |
| 6 | Mô-đun quang SFP 1000BASE-X, TX:1550/1310nm, 20KM trên một sợi quang (Module SFP 1000BASE-X, single Fiber, TX:1550/1310nm, 20KM) | Cái | 2 |  |
| 7 | Mô-đun quang SFP 1000BASE-X, TX:1310/1550nm, 20KM trên một sợi quang (Module SFP 1000BASE-X, single Fiber, TX:1310/1550nm, 20KM) | Cái | 2 |  |
| 8 | Mô-đun quang SFP+ TX1270nm, BIDI, 60km hỗ trợ DDM (Module quang SFP+ TX1270nm, BIDI, 60km support DDM) | Cái | 2 |  |
| 9 | Mô-đun quang SFP+ TX1330nm, BIDI, 60km hỗ trợ DDM (Module quang SFP+ TX1330nm, BIDI, 60km support DDM) | Cái | 2 |  |
| 10 | Mô-đun quang SFP+ 10Ge 10km (Module quang SFP+ 10Ge 10km) | Cái | 2 |  |
| 11 | Mô-đun quang SFP 1.25G, 20km, TX1310/RX1550nm, trên một sợi quang (Module quang SFP 1.25G single fiber, 20km, TX1310/RX1550nm) | Cái | 30 |  |
| 12 | Mô-đun quang SFP 1.25G 20km, TX1550/RX1310nm, trên một sợi quang (Module quang SFP 1.25G single fiber, 20km, TX1550/RX1310nm) | Cái | 30 |  |
| 13 | Thiết bị chuyển mạch switch 28 cổng | Bộ | 2 |  |
| 14 | Thiết bị chuyển đổi tín hiệu quang-điện (SFP sang RJ45) 1GE (Media Converter SFP to RJ45 1GE) | Bộ | 28 |  |
| 15 | Bộ chuyển mạch switch 10G L3 (16 SFP + 8*Combo (GbE/SFP) + 4*10G SFP+), AC+48V DC | Bộ | 2 |  |

- Yêu cầu về kỹ thuật

  - Mô-đun quang XFP TX1270nm, 60km, truyền dẫn hai chiều, hỗ trợ giám sát đo đạc thông số điện tử (Module quang XFP TX1270nm, BIDI, 60km support DDM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | UP to 10.3 Gb/s |
| Type of Plug | 10 Gigabit Small Form Factor Pluggable (XFP) |
| Connector | LC |
| Wavelength | TX1270nm/RX1330nm |
| Max Distance | 60 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun quang SFP+ 10G, TX1330nm, RX1270, 60km, truyền dẫn hai chiều  (Module quang SFP+ 10G, BIDI, 60km (TX1330nm, RX1270nm))

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | UP to 10.3 Gb/s |
| Type of Plug | Enhanced small form-factor pluggable (SFP+) |
| Connector | LC |
| Wavelength | TX1330nm, RX1270nm |
| Max Distance | 60 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun SFP+ 10GE, 850nm, 300m hỗ trợ giám sát đo đạc thông số điện tử (SFP+ 10GE, 850nm, 300m support DDM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | 9.95 ÷ 10.3 Gb/s |
| Type of Plug | Enhanced small form-factor pluggable (SFP+) |
| Connector | Duplex LC |
| Wavelength | 850 nm |
| Max Distance | 300 m |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun XFP 10GE, 850nm, 300m hỗ trợ giám sát đo đạc thông số điện tử (XFP 10GE, 850nm 300m support DDM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | 9.95 ÷ 10.3 Gb/s |
| Type of Plug | 10 Gigabit Small Form Factor Pluggable (XFP) |
| Connector | Duplex LC |
| Wavelength | 850 nm |
| Max Distance | 300 m |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Hộp phối quang 12 cổng (Tối đa 1RU) (kèm dây nhảy, ống nung) (ODF 12 ports (max1RU) (kèm dây nhảy, ống nung))

| Characteristics | Unit |  | Value/Performance |
| --- | --- | --- | --- |
| - Maximum dimension ( Height x Wide ) | mm3 |  | 44.5 x 482 (1U - ODF 12) |
|  |  |  | 89 x 482 (2U - ODF 24) |
|  |  |  | 133 x 482 (3U - ODF48) |
|  |  |  | 266 x 482 (6U - ODF96) |
| - Optical Connector | - |  | SC/APC (Sleeve material: ceramic) |
| - Full accessories | - |  | Yes (pigtail, 60 mm protection fiber tube) |
| - Configuration | - |  | Module or case |
| - Number of connector per case ( or same) | 12 |  | 1 case with 12 adaptors |
|  |  |  | 2 cases with 24 adaptors |
|  |  |  | 4 cases with 48 adaptors |
|  |  |  | 8 cases with 96 adaptors |
| - Same figure or Equivalent | - |  | pictures of inside and outside ODF |

  - Mô-đun quang SFP 1000BASE-X, TX:1550/1310nm, 20KM trên một sợi quang  (Module SFP 1000BASE-X, single Fiber, TX:1550/1310nm, 20KM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | UP to 1.25 Gb/s |
| Type of Plug | Small form-factor pluggable (SFP) |
| Connector | LC |
| Wavelength | TX1550nm/RX1310nm |
| Max Distance | 20 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun quang SFP 1000BASE-X, TX:1310/1550nm, 20KM trên một sợi quang (Module SFP 1000BASE-X, single Fiber, TX:1310/1550nm, 20KM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | UP to 1.25 Gb/s |
| Type of Plug | Small form-factor pluggable (SFP) |
| Connector | LC |
| Wavelength | TX1310nm/RX1550nm |
| Max Distance | 20 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun quang SFP+ TX1270nm, BIDI, 60km hỗ trợ DDM (Module quang SFP+ TX1270nm, BIDI, 60km support DDM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | UP to 10.3 Gb/s |
| Type of Plug | Enhanced small form-factor pluggable (SFP+) |
| Connector | LC |
| Wavelength | TX1270nm/RX1330nm |
| Max Distance | 60 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun quang SFP+ TX1330nm, BIDI, 60km hỗ trợ DDM (Module quang SFP+ TX1330nm, BIDI, 60km support DDM)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | UP to 10.3 Gb/s |
| Type of Plug | Enhanced small form-factor pluggable (SFP+) |
| Connector | LC |
| Wavelength | TX1330nm/RX1270nm |
| Max Distance | 60 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun quang SFP+ 10Ge 10km (Module quang SFP+ 10Ge 10km)

| Characteristics | Value/Performance |
| --- | --- |
| Speed/Data Rate | 9.95 ÷ 10.3 Gb/s |
| Type of Plug | Enhanced small form-factor pluggable (SFP+) |
| Connector | Duplex LC |
| Wavelength | 1310 nm |
| Max Distance | 10 km |
| Compatible | Compatible with existing system CISCO or JUNIPER of SCTV |
| Other | DDM RoHS Hot-swappable |

  - Mô-đun quang SFP 1.25G, 20km, TX1310/RX1550nm, trên một sợi quang (Module quang SFP 1.25G single fiber, 20km, TX1310/RX1550nm)

| SFP 1GE 20km support single core | - Support speed Data link up to 1.25Gbps |
| --- | --- |
|  | - Connector: LC |
|  | - Support Hot-pluggable SFP |
|  | - Maximum distance: 20km |
|  | - Compatible with Cisco, Juniper |
|  | - DDM RoHS |
|  | - Wavelength: TX 1310 nm, RX 1550 nm |

  - Mô-đun quang SFP 1.25G 20km, TX1550/RX1310nm, trên một sợi quang (Module quang SFP 1.25G single fiber, 20km, TX1550/RX1310nm)

| SFP 1GE 20km support single core | - Support speed Data link up to 1.25Gbps |
| --- | --- |
|  | - Connector: LC |
|  | - Support Hot-pluggable SFP |
|  | - Maximum distance: 20km |
|  | - Compatible with Cisco, Juniper |
|  | - DDM RoHS |
|  | - Wavelength: TX 1550 nm, RX 1310 nm |

  - Thiết bị chuyển mạch switch 28 cổng

| STT | CÁC CHỈ TIÊU |  | YÊU CẦU |
| --- | --- | --- | --- |
| I | Yêu cầu phần cứng |  |  |
| 1 | 16 cổng kết nối SFP tốc độ 1G (SFP) 8 port Combo tốc độ 1G (RJ45 or SFP) |  | Có hỗ trợ |
|  | 4x10GbE (SFP+) |  |  |
|  | 1 console port, 1 USB port, 1 RJ45 management port. |  |  |
| 2 | Switching Capacity |  | 128Gbps |
| 3 | Forwarding Rate |  | 95Mpps |
| 4 | Packet buffer |  | 1,5M |
| 5 | Địa chỉ MAC |  | 16K |
| 6 | Multicast MAC address table |  | 4K |
| 7 | Jumbo Frame |  | 10K |
| 8 | Routing Table |  | 1K |
| II | Yêu cầu kỹ thuật |  |  |
| 1 | VLAN |  | Port-based VLAN IEEE802.1Q PVLAN Protocol-base VLAN Voice VLAN MAC VLAN QinQ, Flexible QinQ, VLAN Translation |
| 2 | DHCP |  | DHCP Relay/Option 82 DHCP Snooping/Option 82 DHCP Server/Client for IPv4/IPv6 |
| 3 | Độ tin cậy | Spanning Tree | 802.1D (STP), 802.1W (RSTP), 802.1S (MSTP), Root Guard, BPDU Guard, BPDU Tunnel |
|  |  | LACP | 802.3ad LACP, max 128 group trunks with max 8 ports for each trunk LACP load balance |
|  |  | L2 Ring Protection | ERPS (G.8032) Loopback Detection Fast Link LLDP and LLDP-MED |
|  |  | VSF | Virtual Switch Framework |
| 4 | Multicast |  | IGMP v1/v2/v3 Snooping and L2 Query In Layer 2, such as IGMPv1/v2/v3 snooping and fast leave |
|  |  |  | IGMP v1/v2/v3, IGMP Proxy PIM-DM/SM/SSM Static Multicast Route Multicast Receive Control Multicast VLAN, IGMP v1/v2/v3, IGMP v1/v2/v3 Snooping and L2 Query IPv6 Anycast RP, MLDv1/v2, MLD Snooping Illegal Multicast Source Detect |
|  |  |  | MLD v1/v2 Snooping, Multicast VLAN Register (MVR) |
| 5 | Layer 3 Routing |  | Static Routing Ipv4/ Ipv6, RIPv1/v2, OSPFv2/ OSPFv3, BGP4/ BGP4+ OSPF multiple processes LPM Routing Policy-based routing (PBR) for IPv4 and IPv6 VRRP, URPF, ECMP, BFD |
| 6 | IPv6 |  | 6to4 Tunnel, Configured Tunnel, ISATAP Tunnel, GRE Tunnel ICMPv6, ND, DNSv6 IPv6 LPM Routing, IPv6 Policy-based Routing (PBR) IPv6VRRPv3, IPv6 URPF, IPv6 RA RIPng, OSPFv3, BGP4+ MLD Snooping, IPv6 Multicast VLAN MLDv1/v2, IPv6 Anycast RP, IPv6 ACL, IPv6 QoS |
| 7 | Qos |  | 8 queues per port 802.1p Priority Queues per Port 802.1p Queuing method SWRR, SP, WRR, WDRR, SWDRR Traffic Classification Based on 802.1p COS, ToS, DiffServ DSCP, ACL, port number Traffic Policing PRI Mark/Remark |
| 8 | ACL |  | IP ACL, MACACL, IP-MAC ACL Standard and Expanded ACLBased on source/destination IP or MAC, IP Protocol, TCP/UDP port, DSCP, ToS, IP Precedence), VLAN, Tag/Untag, CoS Redirect and statistics Rules can be configured to port, VLAN, VLAN routing interfaces Time Ranged ACL ACL rules can be configured to port, VLAN |
| 9 | Security |  | 802.1x AAA Port, MAC-based authentication Accounting based on time length and traffic Guest VLAN and auto VLAN RADIUS for IPv4 and IPv6 TACACS+ for IPv4 and IPv6 |
| 10 | Management |  | CLI, WEB, Telnet, SNMPv1/v2c/v3 through IPv4 and IPv6 Syslog and external Syslog Server HTTP SSL SNMP MIB, SNMP TRAP FTP/TFTP SNTP/NTP RMOM 1,2,3,9 Authentication by Radius/TACACS SSH v1/v2 Dual firmware images/ Configuration files 802.3ah OAM, 802.1ag OAM |
| III | Yêu cầu vật lý |  |  |
| 1 | Kích thước |  | Lắp đặt trên Rack 19” |
| 2 | Nguồn cấp |  | 48VDC + 230 V AC |
| 3 | Nhiệt độ làm việc |  | 0°C~50°C, |

  - Thiết bị chuyển đổi tín hiệu quang-điện (SFP sang RJ45) 1GE (Media Converter SFP to RJ45 1GE)

| Features | 10/100Base-TX/1000Base-TX and 1000Base-FX |
| --- | --- |
|  | Ethernet standards IEEE802.3 |
|  | Supported Ports: LC for optical fiber; RJ45 for twisted pair |
|  | Auto-adaptation rate and full/half-duplex mode supported at twisted pair port |
|  | Auto MDI/MDIX supported without need of cable selection |
|  | Up to 6LEDs for status indication of optical power port and UTP port |
|  | Up to 1024 MAC addresses supported |
|  | Optical Port Transmission Rate 1000Mbit/s |
|  | NIC Transmission Rate 10/100/1000Mbit/s |

  - Bộ chuyển mạch switch 10G L3 (16 SFP + 8*Combo (GbE/SFP) + 4*10G SFP+), AC+48V DC

| STT | CÁC CHỈ TIÊU |  | YÊU CẦU |
| --- | --- | --- | --- |
| I | Yêu cầu phần cứng |  |  |
| 1 | 16 cổng kết nối SFP tốc độ 1G (SFP) 8 port Combo tốc độ 1G (RJ45 or SFP) |  | Có hỗ trợ |
|  | 4x10GbE (SFP+) |  |  |
|  | 1 console port, 1 USB port, 1 RJ45 management port. |  |  |
| 2 | Switching Capacity |  | 128Gbps |
| 3 | Forwarding Rate |  | 95Mpps |
| 4 | Packet buffer |  | 1,5M |
| 5 | Địa chỉ MAC |  | 16K |
| 6 | Multicast MAC address table |  | 4K |
| 7 | Jumbo Frame |  | 10K |
| 8 | Routing Table |  | 1K |
| II | Yêu cầu kỹ thuật |  |  |
| 1 | VLAN |  | Port-based VLAN IEEE802.1Q PVLAN Protocol-base VLAN Voice VLAN MAC VLAN QinQ, Flexible QinQ, VLAN Translation |
| 2 | DHCP |  | DHCP Relay/Option 82 DHCP Snooping/Option 82 DHCP Server/Client for IPv4/IPv6 |
| 3 | Độ tin cậy | Spanning Tree | 802.1D (STP), 802.1W (RSTP), 802.1S (MSTP), Root Guard, BPDU Guard, BPDU Tunnel |
|  |  | LACP | 802.3ad LACP, max 128 group trunks with max 8 ports for each trunk LACP load balance |
|  |  | L2 Ring Protection | ERPS (G.8032) Loopback Detection Fast Link LLDP and LLDP-MED |
|  |  | VSF | Virtual Switch Framework |
| 4 | Multicast |  | IGMP v1/v2/v3 Snooping and L2 Query In Layer 2, such as IGMPv1/v2/v3 snooping and fast leave |
|  |  |  | IGMP v1/v2/v3, IGMP Proxy PIM-DM/SM/SSM Static Multicast Route Multicast Receive Control Multicast VLAN, IGMP v1/v2/v3, IGMP v1/v2/v3 Snooping and L2 Query IPv6 Anycast RP, MLDv1/v2, MLD Snooping Illegal Multicast Source Detect |
|  |  |  | MLD v1/v2 Snooping, Multicast VLAN Register (MVR) |
| 5 | Layer 3 Routing |  | Static Routing Ipv4/ Ipv6, RIPv1/v2, OSPFv2/ OSPFv3, BGP4/ BGP4+ OSPF multiple processes LPM Routing Policy-based routing (PBR) for IPv4 and IPv6 VRRP, URPF, ECMP, BFD |
| 6 | IPv6 |  | 6to4 Tunnel, Configured Tunnel, ISATAP Tunnel, GRE Tunnel ICMPv6, ND, DNSv6 IPv6 LPM Routing, IPv6 Policy-based Routing (PBR) IPv6VRRPv3, IPv6 URPF, IPv6 RA RIPng, OSPFv3, BGP4+ MLD Snooping, IPv6 Multicast VLAN MLDv1/v2, IPv6 Anycast RP, IPv6 ACL, IPv6 QoS |
| 7 | Qos |  | 8 queues per port 802.1p Priority Queues per Port 802.1p Queuing method SWRR, SP, WRR, WDRR, SWDRR Traffic Classification Based on 802.1p COS, ToS, DiffServ DSCP, ACL, port number Traffic Policing PRI Mark/Remark |
| 8 | ACL |  | IP ACL, MACACL, IP-MAC ACL Standard and Expanded ACLBased on source/destination IP or MAC, IP Protocol, TCP/UDP port, DSCP, ToS, IP Precedence), VLAN, Tag/Untag, CoS Redirect and statistics Rules can be configured to port, VLAN, VLAN routing interfaces Time Ranged ACL ACL rules can be configured to port, VLAN |
| 9 | Security |  | 802.1x AAA Port, MAC-based authentication Accounting based on time length and traffic Guest VLAN and auto VLAN RADIUS for IPv4 and IPv6 TACACS+ for IPv4 and IPv6 |
| 10 | Management |  | CLI, WEB, Telnet, SNMPv1/v2c/v3 through IPv4 and IPv6 Syslog and external Syslog Server HTTP SSL SNMP MIB, SNMP TRAP FTP/TFTP SNTP/NTP RMOM 1,2,3,9 Authentication by Radius/TACACS SSH v1/v2 Dual firmware images/ Configuration files 802.3ah OAM, 802.1ag OAM |
| III | Yêu cầu vật lý |  |  |
| 1 | Kích thước |  | Lắp đặt trên Rack 19” |
| 2 | Nguồn cấp |  | 48VDC + 230 V AC |
| 3 | Nhiệt độ làm việc |  | 0°C~50°C, |

- Các yêu cầu khác

- Thời hạn bảo hành tối thiểu 01 năm đối với các hàng hóa thuộc phạm vi cung cấp của hợp đồng được tính kể từ khi hàng hóa được bàn giao, nghiệm thu.

- Khuyến khích tăng thời gian bảo hành.

- Bảo hành một đổi một (một thiết bị lỗi đổi một thiết bị mới) miễn phí cho tất cả các vật tư, thiết bị được xác định rằng có những khuyết tật do lỗi cấu trúc hay các lỗi của nhà sản xuất.

- Đảm bảo Hot-line 24/24 trong thời gian bảo hành.

- Cơ chế giải quyết các hư hỏng, khuyết tật phát sinh trong quá trình sử dụng hàng hóa trong thời hạn bảo hành:

+ Chủ đầu tư thông báo cho nhà thầu bằng văn bản, điện thoại, email… về các hư hỏng, khuyết tật phát sinh ngay sau khi xảy ra sự cố.

+ Khi nhận được thông báo của Chủ đầu tư, Nhà thầu phải tiến hành khắc phục các hư hỏng, khuyết tật một cách nhanh chóng nhất có thể và phải cử chuyên gia trực tiếp thực hiện không chậm quá 05 ngày làm việc kể từ khi được yêu cầu. Trong thời gian khắc phục, Nhà thầu sẽ cung cấp thiết bị thay thế hoặc tương đương cho Chủ đầu tư mượn, đảm bảo vận hành được theo yêu cầu của Chủ đầu tư.

+ Trong thời gian bảo hành, Bên Bán chịu trách nhiệm thanh toán các chi phí cho việc khắc phục các hư hỏng, khuyết tật phát sinh do sai sót của Bên Bán. Nếu những sai sót nảy sinh do Bên Mua sử dụng không đúng công năng thì có thể nhờ Bên Bán tiến hành sửa chữa và Bên Mua sẽ thanh toán toàn bộ các chi phí liên quan tới việc sửa chữa đó.

Ghi chú: ngoài việc cung cấp hồ sơ chứng minh tính đáp ứng các thông số/yêu cầu nêu trên, nhà thầu lưu ý cung cấp đủ hồ sơ để đánh giá theo quy định tại Chương III. Tiêu chí đánh giá về kỹ thuật (file đính kèm trên Hệ thống mạng đấu thầu)

Mục 2. Kiểm tra và thử nghiệm hàng mẫu: Không yêu cầu.

* Thông tin liên hệ: Mr. Chung, SĐT: 0989.441.963.