Phần 2. YÊU CẦU VỀ KỸ THUẬT

Chương V. Yêu cầu về kỹ thuật

Yêu cầu về kỹ thuật bao gồm các nội dung cơ bản như sau:

Mục 1. Yêu cầu về kỹ thuật

- Giới thiệu chung về dự án và gói thầu

-	Tên gói thầu: “Mua sắm cáp quang phục vụ hoạt động nhỏ lẻ của các chi nhánh, trung tâm SCTV năm 2025 - đợt 3”.

-	Tên dự toán: Vật tư, thiết bị, nhân công phục vụ hoạt động nhỏ lẻ của các chi nhánh, trung tâm SCTV năm 2025 - đợt 3.

-	Địa điểm thực hiện dự án: Do chủ đầu tư chỉ định (trong khu vực TP Hồ Chí Minh).

-	Hình thức hợp đồng: Hợp đồng trọn gói.

-	Thời gian thực hiện gói thầu: 60 ngày (Trong đó thời gian giao hàng: tối đa 30 ngày kể từ ngày kí hợp đồng).

- Yêu cầu về kỹ thuật

    - Cáp quang 04 sợi FTTx (ngầm) /Cáp quang thuê bao, chôn ngầm: 4 cores đệm lỏng

| Characteristics | Unit | Value/Performance |
| --- | --- | --- |
| FIBER CORE STANDARD | - | ITU-T G652D |
| Fiber Origin | - | Corning/US, LS/Korea or brand from G7/EU |
| Mode field diameter @ 1310nm wavelength | μm | 9.2 ± 0.4 |
| Mode field diameter @ 1550nm wavelength | μm | 10.4 ± 0.5 |
| Core – clad concentricity | μm | ≤ 0.5 |
| Cladding diameter | μm | 125 ± 0.7 |
| Cladding non - circularity | % | ≤ 0.7 |
| Coating diameter | μm | 245 ± 5 |
| Cutoff wavelength | nm | ≤ 1260 |
| Refractive index | - | Ladder form |
| Effective index of refraction Neff (1310nm wavelength) | - | ≤ 1.4677 |
| Effective index of refraction Neff (1550nm wavelength) | - | ≤ 1.4682 |
| Chromatic Dispersion index @ λ 1285 nm - 1330 nm | ps/nm/km | ≤ 3.5 |
| Chromatic Dispersion index @ λ = 1550 nm | ps/nm/km | ≤ 18 |
| Zero Dispersion wavelength | nm | 1310 – 1324 |
| Dispersion slope | ps/nm2/km | ≤ 0.092 |
| Attenuation @ λ = 1310 nm | dB/km | ≤ 0.38 |
| Attenuation @ λ = 1550 nm | dB/km | ≤ 0.24 |
| Fiber color coding | - | EIA/TIA-598 |
| GENERAL CHARACTERISTICS OF OPTICAL CABLE |  |  |
| Standard | - | - Conform TCVN 8665-2011 - Conform IEC and EIA standard - ITU-T G.652.D Standard for core fiber. |
| Cable structure |  |  |
| BUFFER TUBE |  |  |
| Type | - | - Buffer tube is made of loose buffer tube technology. - Loose buffer tube contains, protects single mode fibers and do not affect the movement of fibers inside. |
| Number of fibers per tube | - | 4 |
| Filling compound | - | Buffer tube contains filling compound |
| Water blocking tape | - | Wrap around loose buffer tube, waterproof |
| Reinforce Strength Member (Aramid) | - | Wrap around loose buffer tube and help enhance bearing capacity, less shrinkage, non-conductive |
| INTEGRATED STEEL WIRE | - | - There are at least two steel wires supporting tensile strength, diameter ≥ 0.5 mm/wire, or equivalent structure |
| OUTER JACKET (SHEATH) |  |  |
| Material | - | HDPE (High Density Poly-Ethylen), black |
| Outer sheath thickness (HDPE) | mm | ≥ 1.5 |
| OTHERS |  |  |
| Minimum bending radius | mm | ≤ 180 |
| Sheath marking | - | Engraved name / symbol and sign as required by investors including: name of manufacturer-SCTV, type of cable, month-year of production, length marking in meter. |
| Test specifications certification | - | Cable must have Type Test performed by manufacturer at the factory and confirmed by The Quality Assurance Center of Vietnam. The tests allowed according to the following international IEC / EIA standards or equivalent: * Torsion Test - TIA/EIA-455-85A or IEC-794-1-E7 (IEC 60794-1-2 Method E7) * Tensile & Fiber strain Test - TIA/EIA-455-33A or IEC-794-1-E1 (IEC 60794-1-2 Method E1) * Impact Test - TIA/EIA-455-25C or IEC-794-1-E4 * Flexing Test - TIA/EIA-455-104 or IEC-794-1-E6 * Crush resistance Test - TIA/EIA-455-41A or IEC-794-1-E3 * Bending Test - TIA/EIA-455-33A or IEC-60794-1-E11 * Water penetration Test - TIA/EIA-455-82 or IEC-794-1-F5 * Temperature Test - TIA/EIA-455-3A or IEC-794-1-F1 * Drip Test - IEC-794-1-E14 After test, the optical cable is not broken, sheath is not cracked, growth of attenuation does not exceed 0.1 dB @ 1550nm, waterproof cable. |
| Other requirements: * If necessary, the purchaser has the right to inspect the goods and attenuation tests and type tests at the factory before shipment. * With fiber core, when ordering, request to identification for produced date of fiber core should be within 12 months max. from present, or shipped date. * When delivery goods, fiber cable must be provided attenuation characteristics test report for G.652.D fiber optic at wavelengths: 1310 nm, 1383 nm, 1550 nm and 1625 nm * The usual length of each FTTx 4 cores duct burial cable reel is 2 Km ± 1% (unless other requirements of the investor) * Wooden drum is provided with cable and marked arrows indicate the direction of rotation of the cable reel. |  |  |

    - Cáp quang 12,48,96 sợi chôn ngầm (Cáp quang 12,48,96 core chôn ngầm)

| Characteristics | Unit | Value/Performance |
| --- | --- | --- |
| FIBER CORE STANDARD | - | ITU-T G652D |
| Fiber Origin | - | Corning/US, LS/Korea or brand from G7/EU |
| Mode field diameter @ 1310nm wavelength | μm | 9.2 ± 0.4 |
| Mode field diameter @ 1550nm wavelength | μm | 10.4 ± 0.5 |
| Core – clad concentricity | μm | ≤ 0.5 |
| Cladding diameter | μm | 125 ± 0.7 |
| Cladding non - circularity | % | ≤ 0.7 |
| Coating diameter | μm | 245 ± 5 |
| Cutoff wavelength | nm | ≤ 1260 |
| Refractive index | - | Ladder form |
| Effective index of refraction Neff (1310nm wavelength) | - | ≤ 1.4677 |
| Effective index of refraction Neff (1550nm wavelength) | - | ≤ 1.4682 |
| Chromatic Dispersion index @ λ 1285 nm - 1330 nm | ps/nm/km | ≤ 3.5 |
| Chromatic Dispersion index @ λ = 1550 nm | ps/nm/km | ≤ 18 |
| Zero Dispersion wavelength | nm | 1310 – 1324 |
| Dispersion slope | ps/nm2/km | ≤ 0.092 |
| Attenuation @ λ = 1310 nm | dB/km | ≤ 0.38 |
| Attenuation @ λ = 1550 nm | dB/km | ≤ 0.24 |
| Fiber color coding | - | EIA/TIA-598 |
| GENERAL CHARACTERISTICS OF OPTICAL CABLE |  |  |
| Standard | - | - Conform TCVN 8665-2011 - Conform IEC and EIA standard - ITU-T G.652.D Standard for core fiber. |
| Cable structure |  |  |
| CENTRAL STRENGTH MEMBER (CSM) |  |  |
| Material | - | Non metal FRP (Fibregrass Reinforce with Plastic) |
| Diameter | mm | ≥ 1.5 if cable has less than 48 cores ≥ 2.1 if cable has more than or equal to 48 cores |
| BUFFER TUBE |  |  |
| Type | - | - Buffer tube is made of loose buffer tube technology - Loose buffer tube can be easily separated into sections by specialized tools that do not affect the fiber inside |
| Number of fiber per tube | - | 6 ÷ 12 |
| Color Coding | - | EIA/TIA-598 |
| Filling compound | - | Buffer tubes contains filling compound |
| Stranding | - | SZ stranded around the CSM(FRP), or equivalent |
| JACKET (SHEATH) |  |  |
| Material | - | HDPE (High Density Poly-Ethylen), black |
| Outer sheath thickness (HDPE) | mm | ≥ 1.4 |
| OTHERS |  |  |
| Rip-cord | - | Assembled inside outer sheath (HDPE) and Armor layer (Glass yarn) |
| Water blocking tape | - | Wrap around total loose tubes, waterproof |
| Water blocking yarn | - | Wrap around CSM |
| Outer Strength Member |  | Glass Yarn |
| Armor layer between Outer sheath and Outer Strength Member for waterproof, moisture protection, and rodent species prevention |  | Corrugated steel tape with polymer coating on both side |
| Sheath marking | - | Engraved name / symbol and sign as required by investors including: name of manufacturer- SCTV, type of cable, month-year of production, length marking in meter. |
| Test specifications certification | - | Cable must have Type Test performed by manufacturer at the factory and The Quality Assurance Center of Vietnam confirmed. The tests allowed according to the following international IEC / EIA standards or equivalent: * Torsion Test : - TIA/EIA-455-85A or IEC-794-1-E7 (IEC 60794-1-2 Method E7) * Tensile & Fiber strain Test - IEC-794-1-E1. (IEC 60794-1-2 Method E1) * Impact Test - TIA/EIA-455-25B or IEC-794-1-E4 * Flexing Test - TIA/EIA-455-104 or IEC-794-1-E6 * Crush resistance Test - TIA/EIA-455-41A or IEC-794-1-E3 * Bending Test - IEC-60794-1-E11 * Water penetration Test - TIA/EIA-455-82 or IEC-794-1-F5 * Temperature Test - TIA/EIA-455-3A or IEC-794-1-F1 * Drip Test - IEC-794-1-E14 After test, the optical cable is not broken, sheath is not cracked, growth of attenuation does not exceed 0.1 dB @ 1550nm, waterproof cable. |
| Other requirements: * If necessary, the purchaser has the right to inspect the goods and attenuation tests and type tests at the factory before shipment. * With fiber core, when ordering, request to identification for produced date of fiber core should be within 12 months max. from present, or shipped date. * When delivery goods, fiber cable must be provided attenuation characteristics test report for G.652.D fiber optic at wavelengths: 1310 nm, 1383 nm, 1550 nm and 1625 nm * The usual length of each figure 8 cable reel is 4 Km ± 1% (unless other requirements of the investor) * Wooden drum is provided with cable and marked arrows indicate the direction of rotation of the cable reel. |  |  |

    - Cáp quang hình số 8, dây treo kim loại: 12,24,48 sợi (Cáp quang hình số 8, dây treo kim loại: 12,24,48 core)

| Characteristics | Unit | Value/Performance |
| --- | --- | --- |
| FIBER CORE STANDARD | - | ITU-T G652D |
| Fiber Origin | - | Corning/US, LS/Korea or brand from G7/EU |
| Mode field diameter @ 1310nm wavelength | μm | 9.2 ± 0.4 |
| Mode field diameter @ 1550nm wavelength | μm | 10.4 ± 0.5 |
| Core – clad concentricity | μm | ≤ 0.5 |
| Cladding diameter | μm | 125 ± 0.7 |
| Cladding non - circularity | % | ≤ 0.7 |
| Coating diameter | μm | 245 ± 5 |
| Cutoff wavelength | nm | ≤ 1260 |
| Refractive index | - | Ladder form |
| Effective index of refraction Neff (1310nm wavelength) | - | ≤ 1.4677 |
| Effective index of refraction Neff (1550nm wavelength) | - | ≤ 1.4682 |
| Chromatic Dispersion index @ λ 1285 nm - 1330 nm | ps/nm/km | ≤ 3.5 |
| Chromatic Dispersion index @ λ = 1550 nm | ps/nm/km | ≤ 18 |
| Zero Dispersion wavelength | nm | 1310 – 1324 |
| Dispersion slope | ps/nm2/km | ≤ 0.092 |
| Attenuation @ λ = 1310 nm | dB/km | ≤ 0.38 |
| Attenuation @ λ = 1550 nm | dB/km | ≤ 0.24 |
| Fiber color coding | - | EIA/TIA-598 |
| GENERAL CHARACTERISTICS OF OPTICAL CABLE |  |  |
| Standard | - | - Conform TCVN 8665-2011 - Conform IEC and EIA standard - ITU-T G.652.D Standard for core fiber. |
| Cable structure |  |  |
| CENTRAL STRENGTH MEMBER (CSM) |  |  |
| Material | - | Non metal FRP (Fibregrass Reinforce with Plastic) |
| Diameter | mm | ≥ 1.5 if cable has less than 48 cores ≥ 2.1 if cable has more than or equal to 48 cores |
| BUFFER TUBE |  |  |
| Type | - | - Buffer tube is made of loose buffer tube technology. - Loose buffer tube can be easily separated into sections by specialized tools that do not affect the fiber inside. |
| Number of fiber per tube | - | 6 ÷ 12 |
| Color Coding | - | EIA/TIA-598 |
| Filling compound | - | Buffer tubes contains filling compound |
| Stranding | - | SZ stranded around the CSM(FRP), or equivalent |
| OUTER JACKET (SHEATH) |  |  |
| Material | - | HDPE (High Density Poly-Ethylen), black |
| Outer sheath thickness (HDPE) | mm | ≥ 1.5 |
| INTEGRATED MESSENGER WIRE | - | - If cable has more than or equal to 96 cores: Made of steel wire braided, specifications 7x1mm, covered by PE plastic layer which thickness: ≥1.2mm - If cable has less than or equal to 48 cores: Made of steel wire braided, specifications 7x0.7mm, covered by PE plastic layer which thickness: ≥1.0mm |
| OTHERS |  |  |
| Rip-cord | - | Assembled inside outer sheath (HDPE) |
| Water blocking tape | - | Wrap around total loose tubes, waterproof |
| Water blocking yarn | - | Wrap around CSM |
| Sheath marking | - | Engraved name / symbol and sign as required by investors including: name of manufacturer-SCTV, type of cable, month-year of production, length marking in meter. |
| Test specifications certification | - | Cable must have Type Test performed by manufacturer at the factory and The Quality Assurance Center of Vietnam confirmed. The tests allowed according to the following international IEC / EIA standards or equivalent: * Torsion Test : - TIA/EIA-455-85A or IEC-794-1-E7 (IEC 60794-1-2 Method E7) * Tensile & Fiber strain Test - IEC-794-1-E1. (IEC 60794-1-2 Method E1) * Impact Test - TIA/EIA-455-25B or IEC-794-1-E4 * Flexing Test - TIA/EIA-455-104 or IEC-794-1-E6 * Crush resistance Test - TIA/EIA-455-41A or IEC-794-1-E3 * Bending Test - IEC-60794-1-E11 * Water penetration Test - TIA/EIA-455-82 or IEC-794-1-F5 * Temperature Test - TIA/EIA-455-3A or IEC-794-1-F1 * Drip Test - IEC-794-1-E14 After test, the optical cable is not broken, sheath is not cracked, growth of attenuation does not exceed 0.1 dB @ 1550nm, waterproof cable. |
| Other requirements: * If necessary, the purchaser has the right to inspect the goods and attenuation tests and type tests at the factory before shipment. * With fiber core, when ordering, request to identification for produced date of fiber core should be within 12 months max. from present, or shipped date. * When delivery goods, fiber cable must be provided attenuation characteristics test report for G.652.D fiber optic at wavelengths: 1310 nm, 1383 nm, 1550 nm and 1625 nm * The usual length of each figure 8 cable reel is 4 Km ± 1% (unless other requirements of the investor) * Wooden drum is provided with cable and marked arrows indicate the direction of rotation of the cable reel. |  |  |

    - Cáp thuê bao 2 sợi đệm chặt (Cáp thuê bao 2FO):

| Characteristics | Unit | Value/Performance |
| --- | --- | --- |
| FIBER CORE STANDARD | - | ITU-T G652D |
| Fiber Origin | - | Corning/US, LS/Korea or brand from G7/EU |
| Mode field diameter @ 1310nm wavelength | μm | 9.2 ± 0.4 |
| Mode field diameter @ 1550nm wavelength | μm | 10.4 ± 0.5 |
| Core – clad concentricity | μm | ≤ 0.5 |
| Cladding diameter | μm | 125 ± 0.7 |
| Cladding non - circularity | % | ≤ 0.7 |
| Coating diameter | μm | 245 ± 5 |
| Cutoff wavelength | nm | ≤ 1260 |
| Refractive index | - | Ladder form |
| Effective index of refraction Neff (1310nm wavelength) | - | ≤ 1.4677 |
| Effective index of refraction Neff (1550nm wavelength) | - | ≤ 1.4682 |
| Chromatic Dispersion index @ λ 1285 nm - 1330 nm | ps/nm/km | ≤ 3.5 |
| Chromatic Dispersion index @ λ = 1550 nm | ps/nm/km | ≤ 18 |
| Zero Dispersion wavelength | nm | 1310 – 1324 |
| Dispersion slope | ps/nm2/km | ≤ 0.092 |
| Attenuation @ λ = 1310 nm | dB/km | ≤ 0.38 |
| Attenuation @ λ = 1550 nm | dB/km | ≤ 0.24 |
| Fiber color coding | - | EIA/TIA-598 |
| GENERAL CHARACTERISTICS OF OPTICAL CABLE |  |  |
| Standards | - | - Conform TCVN 8665-2011 - Conform IEC and EIA standard - ITU-T G.652.D Standard for core fiber. |
| Cable structure |  |  |
| Type | - | Tight buffer |
| Number of fibers | - | 2 |
| Cross – sectional dimension (without hanging wire) | mm | 2.0 x 3.0 ± 0,1 mm |
| Outer sheath |  |  |
| Material | - | PVC (PolyVinyl Chloride) or LSZH (Low Smoke Zero Halogen) or PE (Poly-Ethylen), black |
| Thickness | mm | ≥ 0.6 |
| Features | - | - UV – Resistant. - Contains suitable antioxidant. - Does not develop mildew. - Electrical insulation. - Protects the core from mechanical and environmental forces during storage, assembly and operation (water, heat, chemicals, insect bites, …). - Subscriber line coating is smooth, concentric, without connecting points, without cracks, without holes, of even quality ( not rough, does not have bubbles, not divided, no bumps, no holes, no lumps), does not contain metal; flexible, firm, easy to open; optical fiber does not stick to coating when opening; sleeve thickness is even along cable length. |
| Hanging wire | - |  |
| Material | - | zinc – coated steel wire |
| Diameter | mm | 1 wire x ≥ 1 ± 0,1 Or 7 wires x ≥ 0,33 ± 0,01 |
| Bearing wire (Fiber protect steel) |  |  |
| Material | - | zinc – coated steel wire or FRP |
| Number of wire | - | 2 – Placed on both sides to protect the optical fiber |
| Diameter | mm | 0.5 ± 0.05 mm |
| Others |  |  |
| Minimum bending radius | mm | ≤ 180 |
| Sheath marking | - | Printed or engraved name / symbol and sign as required by investors including: name of manufacturer-SCTV, type of cable, month-year of production, length marking in meter. |
| Test specifications certification | - | Cable must have Type Test performed by manufacturer at the factory and confirmed by The Quality Assurance Center of Vietnam. The tests allowed according to the following international IEC / EIA standards or equivalent: * Torsion Test - TIA/EIA-455-85A or IEC-794-1-E7 (IEC 60794-1-2 Method E7) * Tensile & Fiber strain Test - TIA/EIA-455-33A or IEC-794-1-E1 (IEC 60794-1-2 Method E1) * Impact Test - TIA/EIA-455-25C or IEC-794-1-E4 * Flexing Test - TIA/EIA-455-104 or IEC-794-1-E6 * Crush resistance Test - TIA/EIA-455-41A or IEC-794-1-E3 * Bending Test - TIA/EIA-455-33A or IEC-60794-1-E11 * Water penetration Test - TIA/EIA-455-82 or IEC-794-1-F5 * Temperature Test - TIA/EIA-455-3A or IEC-794-1-F1 After test, the optical cable is not broken, sheath is not cracked, growth of attenuation does not exceed 0.1 dB @ 1550nm, waterproof cable. |
| Other requirements: * If necessary, the purchaser has the right to inspect the goods and attenuation tests and type tests at the factory before shipment. * With fiber core, when ordering, request to identification for produced date of fiber core should be within 12 months max. from present, or shipped date. * When delivery goods, fiber cable must be provided attenuation characteristics test report for G.652.D fiber optic at wavelengths: 1310 nm, 1383 nm, 1550 nm and 1625 nm * The usual length of each FTTx 2 cores duct burial cable reel is 1 Km ± 1% (unless other requirements of the investor) * Wooden or plastic drum is provided with cable and marked arrows indicate the direction of rotation of the cable reel. |  |  |

- Các yêu cầu khác

- Thời hạn bảo hành tối thiểu 5 năm đối với cáp quang và 3 năm đối với cáp thuê bao thuộc phạm vi cung cấp của hợp đồng được tính kể từ khi hàng hóa được bàn giao, nghiệm thu.

- Khuyến khích tăng thời gian bảo hành.

- Bảo hành một đổi một (một cuộn cáp lỗi đổi một cuộn cáp mới) miễn phí cho tất cả các vật tư, thiết bị được xác định rằng có những khuyết tật do lỗi cấu trúc hay các lỗi của nhà sản xuất.

- Đảm bảo Hot-line 24/24 trong thời gian bảo hành.

- Cơ chế giải quyết các hư hỏng, khuyết tật phát sinh trong quá trình sử dụng hàng hóa trong thời hạn bảo hành:

+ Chủ đầu tư thông báo cho nhà thầu bằng văn bản, điện thoại, email về các hư hỏng, khuyết tật phát sinh ngay sau khi xảy ra sự cố.

+ Khi nhận được thông báo của Chủ đầu tư, Nhà thầu phải tiến hành khắc phục các hư hỏng, khuyết tật một cách nhanh chóng nhất có thể và phải cử chuyên gia trực tiếp thực hiện không chậm quá 05 ngày làm việc kể từ khi được yêu cầu.

+ Trong thời gian bảo hành, Bên Bán chịu trách nhiệm thanh toán các chi phí cho việc khắc phục các hư hỏng, khuyết tật phát sinh do sai sót của Bên Bán. Nếu những sai sót nảy sinh do Bên Mua sử dụng không đúng công năng thì có thể nhờ Bên Bán tiến hành sửa chữa và Bên Mua sẽ thanh toán toàn bộ các chi phí liên quan tới việc sửa chữa đó.

Ghi chú: ngoài việc cung cấp hồ sơ chứng minh tính đáp ứng các thông số/yêu cầu nêu trên, nhà thầu lưu ý cung cấp đủ hồ sơ để đánh giá theo quy định tại Chương III. Tiêu chí đánh giá về kỹ thuật (file đính kèm trên Hệ thống mạng đấu thầu)

Mục 2. Kiểm tra và thử nghiệm hàng mẫu: Không yêu cầu.

* Thông tin liên hệ: Ms. Vũ Hải, SĐT: 0914 960 882.