EXHIBIT I

SCOPE OF WORK

for

PROVISION OF FAULT SEAL ANALYSIS FOR DAI HUNG FIELD”

BLOCKS 05-1(a) - NAM CON SON BASIN

OFFSHORE VIET NAM

# INTRODUCTION

Block 05-1(a) is located in the north-western part of Nam Con Son basin, distance approximately 265 km South-East to Vung Tau City (Figure 1). The water depth is various form 110 to 120m. The total ares of the Block is about 535km2.

Figure 1: Block 05-1(a) proposed study areas

The Dai Hung Field is currently producing from major fault blocks within the Lower Miocene clastic and Middle Miocene carbonate. The complex geological and tectonic settings, characterized by multiple faulting phases, make fault seal evaluation critical to:

- Clarify hydrocarbon trapping mechanisms and remaining reserves distribution.

- Support decisions for exploration expansion to adjacent structural blocks such as Than Nong, X1, X2, X3, Southern and Southwestern areas.

- Integrate with ongoing studies such as Seismic Inversion, AVO analysis, and production data.

- Contribute to risk assessment and wells design.

- Enhance success probability for future exploration wells during 2026–2030 period.

Previous fault seal evaluations have not been updated with new well data, lacked quantification, and didn't effectively visualize fault sealing properties. Therefore, an advanced study using multiple data sources and advanced techniques is required to improve the prospectivity of future exploration wells.

# OBJECTIVES

The objectives of study “Fault seal of Dai Hung field” are listed below:

- Quantitative evaluation of the sealing capacity of various fault types in the drilled area to determine hydrocarbon potential and minimize risk.

- Build a 3D model to determine pressure differential across faults and assess controlling factors such as SGR, juxtaposition, etc.

- Classify faults based on seal quality (good, moderate, poor)

- Quantitatively assess seal potential of faults in undrilled areas within and around the field

- Provide technical recommendations for exploration and drilling decisions.

The study is conducted within the Dai Hung field and adjacent areas, covering the entire geological structure and fault network within the existing 3D seismic data coverage and wells operated by PVEP-NCS.

- Drilled blocks: Analyze and evaluate seal for at least 20 faults in areas with existing wells as a basis for application to undrilled targets.

- Undrilled blocks: Focus on X1, X3, X4, X5 fault block, then extension to the south and southwest of Dai Hung field (Than Nong area, western downthrown blocks) and other internal fault blocks identified in the current Petrel model, that is interested in expanding exploration activities by PVEP-NCS.

# DATABASE

The data use for this study provided by PVEP-NCS, include but are not limited to:

- 3D seismic data (existing 3D cubes in block 05-1(a), seismic data interpretation, seismic attributes).

- Well data (logs, formation pressure).

- Fluid analysis reports (PVT, oil density, GOR).

- Structural maps and regional geological reports.

- Existing Petrel geological model.

- Data QC and consistency checks.

- Other information necessary for Service execution as required by Contractor. (As required).

All data to be provided on DVD, USB disks or via email/ftp communication.

# TECHNICAL SCOPE OF STUDY REQUIREMENT

This scope outlines the advanced technical requirements for Services at the Đại Hùng fields, offshore Vietnam, which the CONTRACTOR will provide to the COMPANY. The scope includes, but is not limited to, the following minimum requirements:

- The services rendered by the CONTRACTOR must adhere fully to the COMPANY's specifications as well as relevant national and international standards.

- The CONTRACTOR will be responsible for the delivery of services. While the CONTRACTOR may engage Subcontractors to perform parts of the services, both the CONTRACTOR and any Subcontractors must operate under the supervision of the COMPANY.

- The CONTRACTOR is required to supply all personnel, transportation, and other essentials needed to complete the job according to the COMPANY's requirements.

- Any additional tasks outside the defined scope of work will be the responsibility of the CONTRACTOR. The CONTRACTOR will not bill the COMPANY for any costs associated with these extra tasks. The total cost will be covered by a lump sum agreement.

The following describes the basic scope whilst not meant to be prescriptive, should be used as a guide for the basic study. Input to the tender in this Section will form part of the technical evaluation criteria for study, the more detailed scope, and the higher score in technical evaluation. The workflow of study includes General steps and two phases for Quality Control, detail as below.

- GENERAL STEPS

  - Database Collection and Review

- Review seismic interpretation including fault system, horizon maps; structural maps; carry out the re-interpretation and re-mapping if needed.

- Review well data (log, reservoir pressure)

- Summarize, quality assessment, and enhancement of geological and geophysical datasets.

- Review Fluid analysis reports (PVT, oil density, GOR).

- Review Pressure data from MPLT/ RCI, production data.

- Review Existing Petrel geological modes.

- Review and re-interpretation of well log data as required.

  - Determine Rule of Reservoir Pressure Distribution

- Analyze/ Evaluate pressure gradient trends in wells (every fault block and depth).

- Forecast reservoir pressures in wells.

- Identify the start point of abnormal pressure in wells.

- Construct 2D pressure cross-sections distribution through wells, faults. On that basis, generate visual pressure distribution maps for areas with predicted pressure.

  - Fault Characteration

Select representative faults for detailed description.

  - 3D Structural, Fault, and Lithology Modeling

- Build a 3D structural model (structure, faults, lithology).

- Evaluate the pressure difference across the fault plane between two points at the same depth or between two adjacent grid cells on opposite sides of the fault.

- Calculate fault seal models (At least 2 models below and propose additional models suitable for Dai Hung field).

- Build 2 separate model:

- Juxtaposition Analysis

- Calculating Shale Gouge Ratio (SGR) on the fault surface, using 2 separate lithology models: VCL from well logs and VCL from seismic inversion results.

  - Determine the pressure distribution for individual according to dynamic model

PVEP-NCS has been developing simulation models for production. Based on these models, the bidders will review, extract data, and generate pressure distribution maps when wells were drilled (1994, 2003, 2010, 2014, 2024 and 2025), and create maps showing pressure changes relative to the initial time point.

Review production wells that penetrate into new blocks and assess the pressure difference across typical reservoirs to evaluate the sealing capacity of related faults. Adjust model parameters if necessary.

The 2 phases of the study are divided based on priority level for the exploration expansion.

- PHASE I: Fault Seal Capacity Evaluation in the fault block X1 and adjacent blocks X3, X4, X5

- Visualize fault sealing capability through images/models, SGR values, and pressure differentials across the fault

- For each layer, each group of layers (reservoir) in each fault block, extract data from the weakest point.

- Cross check with the presence/absence of fluid contacts and adjust quantitative parameters accordingly.

- Apply to faults in structures. Predict boundaries for targets based on the model. Apply fault sealing for faults in the fault block X1 and adjacent blocks X3, X4, X5.

- Conclusion, risk evaluation, and recommendation of exploration drilling

- PHASE II: Fault sealing capability evaluation for remaining blocks up to the western downthrown blocks, Thần Nông area.

- Visualize fault sealing capability through images/models, SGR values, and pressure differentials across the fault

- For each layer, each group of layers (reservoir) in each fault block, extract data from the weakest point.

- Cross check with the presence/absence of fluid contacts and adjust quantitative parameters accordingly.

- Apply to faults in structures. Predict boundaries for targets based on the model. Apply fault sealing for remaining blocks up to the western downthrown blocks, Thần Nông area.

- Conclusion, risk evaluation, and recommendation of exploration drilling.

# DURATION TIME OF THE STUDY:

The service is scheduled to commence and complete in around 06 months starting from confirmed receipt of complete dataset and shall be discussed in details to be fit with requirements and schedule of Block 05-1(a).

BIDDER is requested to propose schedule of work for this project and specific steps in the Overall Project Schedule chart. BIDDER is also request highlight QC times in Overall Project Schedule.

# CONTRACTOR EXPERIENCES:

BIDDER is kindly requested to provide list of similar projects which has been carried out by Bidder in last 05 years. Having track record study or similar technology was done in Nam Con Son Basin –Offshore Vietnam for COMPANY’s reference will be preferable.

BIDDER is requested to:

- Provide a sufficient number of personnel for each position/work requested to perform the WORK.

- Provide details particular such as curriculum vitae (CV), qualification, years of experience, specify projects had been participated in, stating the position classification and training, ethnic background for the respective personnel for the performance of the WORK.

- Be responsible for all necessary arrangements required in connection with the issuance of work permit, visa, etc. for its personnel engaged for the WORK (if any);

- BIDDER has the right to submit proposal for a greater number of personnel than mentioned below for COMPANY required. COMPANY shall have at its option, the right to reject any personnel proposed by BIDDER;

- BIDDER shall also ensure that the personnel provided have adequate command of the English language.

- BIDDER shall keep its personnel for the exclusive use of COMPANY during the execution of the WORK.

- BIDDER is requested to provide COMPANY the list of Senior Geophysicists, Senior Geologists & Senior Petrophysicist together with their details experience and list of projects that they have executed in the past.

- Team lead:

- At least 15 years’ experience in petroleum industry; at least 05 years experienced in proposal position. Having track record on similar project is preferable.

- Senior Geophysicists, Senior Geologists, Senior Petrophysicist:

- At least 10 years’ experience in petroleum industry; at least 05 years experienced in proposal position. Having track record on similar project is preferable.

- Other Geophysicist, Geologist, Petrophysicist with minimum 05 years’ experience in petroleum industry.

- All the personnel experience projects must be listed down in details with project names, project titles & year done in the personnel CVs.

# HARDWARE AND SOFTWARE:

- All software/licenses along with hardware are to be provided by the CONTRACTOR.

- All software must be commercial.

# SERVICE LOCATION

- The service location will be carried out in CONTRACTOR’S study location. CONTRACTOR will specify the service location.

# FINAL DATA DELIVERY

CONTRACTOR shall ensure delivery to COMPANY upon completion of the work or in accordance with COMPANY's instruction the following data:

- Detailed fault seal analysis report

Prepare a detailed report, including:

- Methodology for evaluating fault seal capacity.

- Characteristics of fault seals (based on evaluation from structural models, lithology, and pressure...) in areas with existing wells. Basis for classifying fault seal types.

- Evaluation of fault seal capacity in undrilled structures. Risk level of the evaluation.

- Present the research results.

- Distribution maps.

- 3D models.

- Excel files with extracted fault surface data.

The COMPANY reserves the right to request for all and any reasonable reports and deliverables other than those as specified in table above which will be used for further processing and data interpretation purpose.

The deliverables are included in the costs quoted.

# WORKSHOP, QC AND MEETING

- CONTRACTOR is requested to organize online meeting bi-weekly and submit report with detailed results. PVEP NCS may give comments & suggest adjustments on the current work or work progress.

- Frequent discussion or meeting when a new step starts and when required. Conduct frequent discussion with operator to ensure all assumptions is supported by right agreement with MOM recorded;

- At least 04 technical workshop offline (01 KO meeting, 02 QC, 01 final workshop) & with COMPANY/PVEP/PVN. Organizing offline meeting: CONTRACTOR arrangement for COMPANY’s personnel attending the meeting (travelling & lodging) at Bidder’s cost.

# CONFIDENTIALITY

COMPANY and BIDDER staff working on the projects will ensure strict confidentiality of all data; information and results used and obtained in the project undertaken, and not disclose these to any third party.