EXHIBIT I

SCOPE OF WORK

for

PROVISION SERVICE OF STRUCTURAL ZONATION STUDY, UPDATE HYDROCARBON POTENTAIL AND SELECTION FOR NEXT EXPLORATION PROSPECTS/WELLS

BLOCKS 05-1(a) - NAM CON SON BASIN

OFFSHORE VIET NAM

# INTRODUCTION

Block 05-1(a) is located in the north-western part of Nam Con Son basin, distance approximately 265 km South-East to Vung Tau City (Figure 1). The water depth is various form 110 to 120m. The total ares of the Block is about 535km2.

Figure 1: Block 05-1(a) proposed study areas

Block 05-1(a) has been strongly affected by Miocene tectonic activities, which resulted in the development of complex geological structures, predominantly in the form of fault-bounded blocks and structural highs associated with major fault systems.

In previous studies, structural interpretation was mainly focused on the Dai Hung field area (DH01, DH02), due to limitations in seismic and geological data coverage outside the field. In marginal areas between different seismic surveys, data quality was degraded by limited fold coverage, migration noise (swing migration), and time mismatches caused by differences in acquisition parameters among adjacent Blocks.

To overcome these limitations, in 2025 the Operator (PVEP-NCS) implemented a 3D seismic reprocessing project covering nearly 800 km² of seismic data acquired in 1991 and 2013 within Block 05-1(a), together with selected seismic data from adjacent Blocks 04-3 and 05-1b&c (Figure 2). The data were reprocessed using Pre-Stack Depth Migration (PSDM). This approach ensures optimal fold coverage at block boundaries and significantly improves seismic imaging quality.

The reprocessed seismic dataset provides the basis for constructing an integrated structural framework for the entire Block 05-1(a) and establishing structural continuity with adjacent Blocks using a unified seismic dataset. This enables a clearer understanding of the structural–tectonic characteristics of the area, delineation of hydrocarbon-bearing prospects, and ranking of exploration opportunities. These results form a critical basis for defining future exploration strategies of the Operator.

Figure 2: 800 km² of re-processed 3D seismic data (PSDM 2025) Block 05-1(a)

# OBJECTIVES

The objectives of study “Structural zonation, hydrocarbon potential update and selection of next exploration prospect/ wells” based on approximately 800sq.km of 2025 seismic data (Figure 02), are to:

- Develop a comprehensive and internally consistent tectonic and structural framework for the study area based on reprocessed high-quality 3D PSDM seismic data;

- Establish a consistent and integrated structural zonation for the area.

- Update and re-evaluate hydrocarbon potential for identified prospects.

- Assess geological Probability of Success (POS), select prioritized prospects, and propose exploration well locations to support exploration planning and subsequent field development activities.

# DATABASE

The data use for this study provided by PVEP-NCS, include but are not limited to:

- 2025 reprocessed 3D PSDM seismic data: ~800 km².

- Seismic velocities data.

- Latest updated interpretation, structural map, fault (polygon and fault stick), thickness maps for both time and depth domains.

- Well data: All available well data in block 05-1(a) including MWD data, Master logs, wireline logs, composite logs, mud logs, petrophysical analysis reports, geochemical reports, core sample analysis, sidewall sample analysis, cutting samples, FMI interpretation, DST reports, production data, ...

- Geological Data and Other Reports

- Other information necessary for Service execution as required by Contractor. (As required).

All data to be provided on DVD, USB disks or via email/ftp communication.

# TECHNICAL SCOPE OF STUDY REQUIREMENT

This scope outlines the advanced technical requirements for Services at the DH fields, offshore Vietnam, which the CONTRACTOR will provide to the COMPANY. The scope includes, but is not limited to, the following minimum requirements:

- The services rendered by the CONTRACTOR must adhere fully to the COMPANY's specifications as well as relevant national and international standards.

- The CONTRACTOR will be responsible for the delivery of services. While the CONTRACTOR may engage Subcontractors to perform parts of the services, both the CONTRACTOR and any Subcontractors must operate under the supervision of the COMPANY.

- The CONTRACTOR is required to supply all personnel, transportation, and other essentials needed to complete the job according to the COMPANY's requirements.

- Any additional tasks outside the defined scope of work will be the responsibility of the CONTRACTOR. The CONTRACTOR will not bill the COMPANY for any costs associated with these extra tasks. The total cost will be covered by a lump sum agreement.

The following describes the basic scope whilst not meant to be prescriptive, should be used as a guide for the basic study. Input to the tender in this Section will form part of the technical evaluation criteria for study, the more detailed scope, and the higher score in technical evaluation. The workflow of study is comprised in generally in below tasks for Quality Control:

- Database Collection, QC

- Review seismic interpretation including fault system, horizon maps; carry out QC and the re-interpretation, remapping if needed.

- Compilation, quality assessment, and enhancement of geological and geophysical datasets.

- Review and re-interpretation of well log data as required.

- Tectonic Interpretation and Structural Zonation

- Reconstruction of the tectonic evolution of the Nam Con Son Basin through major geological stages (rifting/extension, thermal subsidence, inversion/uplift), and evaluation of its control on sedimentation and structural development;

- Identification and characterization of dominant tectonic regimes (extensional, compressional, strike-slip, or combined), including their spatial distribution and temporal evolution;

- Interpretation of characteristic seismic reflectors, reflector geometry changes, and amplitude displacements related to tectonic activity;

- Identification of major structural styles such as folds, faults, horsts and grabens, and analysis of stress–strain regimes controlling present-day structures;

- Detailed analysis of major and subsidiary fault systems based on fundamental geological principles (superposition, original horizontality, and cross-cutting relationships);

- Construction of structural maps for key stratigraphic horizons.

- Integrate seismic data, maps, wells, and specialized software to simulate 3D structures and create geological cross-sections to clarify the structure.

- Refer to outcrop data analysis or itc … to identify structural elements.

- Delineation of major structural domains and evaluation of structural closure, trap geometry, areal extent, and structural uncertainty.

- Evaluation of the impact of tectonic phases on trap formation, trap destruction capability, and hydrocarbon preservation;

- Overall assessment of the petroleum system for each structural zone based on integrated tectonic, geological, and geophysical results

- Hydrocarbon Potential Update and Selection of Exploration Prospects/Wells

- Delineation of prospective structures/ faulted blocks based on structural zonation, closure mapping, and petroleum system elements;

- Detailed structural evaluation of each prospect, including trap geometry, area, lateral and vertical closure, fault behavior…;

- Estimation and update of hydrocarbon resources for each prospect based on integrated well results and geological interpretation;

- Geological risk analysis and petroleum system evaluation, including source rock presence and maturity, hydrocarbon generation and migration, reservoir quality, seal integrity, and timing of trap formation versus charge;

- Ranking of prospects based on integrated criteria including resource, geological risk, and technical feasibility;

- Selection of prioritized prospects and recommendation of exploration well targets;

- Definition of future exploration strategy and recommendations for additional data acquisition or studies if required

# DURATION TIME OF THE STUDY:

The service is scheduled to commence and complete in around 06 months starting from confirmed receipt of complete dataset and shall be discussed in details to be fit with requirements and schedule of Block 05-1(a).

BIDDER is requested to propose schedule of work for this project and specific steps in the Overall Project Schedule chart. BIDDER is also request highlight QC times in Overall Project Schedule.

# CONTRACTOR EXPERIENCES:

BIDDER is kindly requested to provide list of similar projects which has been carried out by Bidder in last 05 years. Having track record study or similar technology was done in Nam Con son Basin –Offshore Vietnam for COMPANY’s reference will be preferable.

BIDDER is requested to:

- Provide a sufficient number of personnel for each position/work requested to perform the WORK.

- Provide details particular such as curriculum vitae (CV), qualification, years of experience, specify projects had been participated in, stating the position classification and training, ethnic background for the respective personnel for the performance of the WORK.

- Be responsible for all necessary arrangements required in connection with the issuance of work permit, visa, etc. for its personnel engaged for the WORK (if any);

- BIDDER has the right to submit proposal for a greater number of personnel than mentioned below for COMPANY required. COMPANY shall have at its option, the right to reject any personnel proposed by BIDDER;

- BIDDER shall also ensure that the personnel provided have adequate command of the English language.

- BIDDER shall keep its personnel for the exclusive use of COMPANY during the execution of the WORK.

- BIDDER is requested to provide COMPANY the list of Senior Geophysicists, Senior Geologists & Senior Petrophysicist together with their details experience and list of projects that they have executed in the past.

- Team lead:

- At least 15 years’ experience in petroleum industry; at least 05 years experienced in proposal position. At least, the personnel must be done 03 similar project or similar work scope.

- Senior Geologists, Senior Geophysicists, Senior Petrophysicist:

- At least 10 years’ experience in petroleum industry; at least 05 years experienced in proposal position. At least, the personnel must be done 03 similar project or similar work scope.

- Other Geophysicist, Geologist, Petrophysicist with minimum 5 years’ experience in petroleum industry.

- All the personnel experience projects must be listed down in details with project names, project titles & year done in the personnel CVs.

# HARDWARE AND SOFTWARE:

- All software/licenses along with hardware are to be provided by the CONTRACTOR.

- All software must be commercial. Bidder is requested to provide suitable software to perform detail seismic interpretation. The output results should be input/show and use on Petrel Project.

# SERVICE LOCATION

- The service location will be carried out in CONTRACTOR’S study location. CONTRACTOR will specify the service location.

# FINAL DATA DELIVERY

CONTRACTOR shall ensure delivery to COMPANY upon completion of the work or in accordance with COMPANY's instruction the following data:

- Structural maps, hydrocarbon potential maps, and representative seismic sections;

- Ranked list of all identified prospects;

- Detailed interpretative maps of tectonic elements and evolution (structures, faults, folds, deformation, etc.) for the main horizons;

- Updated hydrocarbon resource estimates and geological POS assessments;

- Recommended exploration prospects and well targets;

- Digital interpretation datasets (seismic interpretation, maps, figures) compatible with Petrel;

- Final technical report (DOCX and PDF formats);

- Presentation materials.

The COMPANY reserves the right to request for all and any reasonable reports and deliverables other than those as specified in table above which will be used for further processing and data interpretation purpose.

The deliverables are included in the costs quoted.

# WORKSHOP, QC AND MEETING

- CONTRACTOR is requested to organize online meeting bi-weekly and submit report with detailed results. PVEP NCS may give comments & suggest adjustments on the current work or work progress.

- Frequent discussion or meeting when a new step starts and when required. Conduct frequent discussion with operator to ensure all assumptions is supported by right agreement with MOM recorded;

- At least 03 technical workshop offline & with COMPANY/PVEP/PVN. Organizing offline meeting: CONTRACTOR arrangement for COMPANY’s personnel attending the meeting (travelling & lodging) at Bidder’s cost.

# CONFIDENTIALITY

COMPANY and BIDDER staff working on the projects will ensure strict confidentiality of all data; information and results used and obtained in the project undertaken, and not disclose these to any third party.