EXHIBIT I

SPECIAL PROVISION AND SCOPE OF WORKS

| TABLE OF CONTENTS |  |
| --- | --- |
| SECTION | TITLE |
| I | GENERAL INFORMATION |
| II | SPECIAL PROVISIONS |
| III | REQUEST FOR SERVICES |
| IV V | SCOPE OF WORK CONTRACTOR WORK PERFORMANCE EVALUATION |

I	GENERAL INFORMATION

- AREA OF OPERATIONS:	Block X (Open water), Block 05.1a; Offshore Vietnam

- WELL NAME	:		DH-18X, DH-19X

- WELL TYPE:			Exploration/DST

- START TIME:	 Tentative in August 2026 for DH-18X and May 2027 for DH-19X

- ESTIMATED TIME:		122 days

- WATER DEPTH:		+/-109m

- RIG TYPE:			Jack up 400ft WD

- AREA OF OPERATIONS: 	WHP-DHN, Block 05.1a; Offshore Vietnam.

- 1st Period WELL NAME:

- Firm: DHN-1X, DHN-2X, DHN-4X, DHN-1P, DHN-2P.

- Optional: 2 Exploration wells ( DH-18X & DH-19X).

- 2nd Period WELL NAME:

- Firm: DHN-3P, DHN-4PI, DHN-5PI, DHN-6PI.

- Optional: 2 Exploration wells ( DH-20X & DH-21X).

- WELL TYPE:

- Tie-back + Production + Production/Injection wells for Firm wells.

- Exploration for Optional wells.

- START TIME:

- 1st Period: 		Tentative on 15 October 2026.

- 2nd Period: 		Tentative on 9 July 2027.

- ESTIMATED TIME:

- 1st Period: 	177 days for Firm wells + 127 days for Optional wells.

- 2nd Period: 	171 days for Firm wells + 112 days for Optional wells.

- WATER DEPTH:		+/-109m.

- RIG TYPE:			Jack up 400ft WD.

- DELIVERY MODE:		“Call-out” basis.

- Well information

  - Information for the Firm wells

Total of 9 Wells: 3 tie-back/production, 3 production and 3 injection/production wells with well Targets are at Thông – Mang Cau formation (Carbonate) from H44 – H76.2.

  - Information for the Optional wells

| Well Classification | Location | Well Quantity | Period | Note |
| --- | --- | --- | --- | --- |
| Exploration | Open Water – DH Field | 4 | 2 wells at 1st Period 2 wells at 2nd Period | Drill & DST |

DH-18X (1st Period)

Exploration well with Targets are at Thong – Mang Cau formation (Carbonate) and Dua formation (Clastic).

| Location | Well name / Type |  | Slot | Note |
| --- | --- | --- | --- | --- |
| Block 05-1(a) | DH-18X | Exploration | Open Water | Drill & DST for Clastic reservoir |

- Well Structure

  - Well Structure for Production/Injection Well

26” hole/20” conductor x 16” hole/13-3/8” casing x 12-1/4” hole/9-5/8” casing.

  - Exploration Well Structure

26” hole/20” conductor x 16” hole/13-3/8” casing x 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner.

  - Well Structure for Sidetrack wells

Tie-back 20” Conductor & 13-3/8” Casing. Cut 9-5/8” casing & Sidetrack 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner.

  - Structure for Tie-back Wells

- DHN-4X and DHN-2X: Tie-back 20” Conductor, 13-3/8” Casing, 9-5/8” Casing from seabed to surface.

- DHN-1X: Tie-back 20” Conductor, 9-5/8” Casing from seabed to surface. Tie-back 7” from +/-3,257m to surface.

2.5 Structure for DH-18X

- 26” hole/20” conductor x 16” hole/13-3/8” casing x 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner casing

- Well lay out and drilling schedule

11 Slots: 2 slots (Dual for future wells – Blue color) + 9 slots x 20” Conductor.

- Basic Geological well information

The well is expected normal temperature and normal pore pressure.

  - Lithology DHN

  - Lithology DH-18X

  - Pressure, Temperature Profiles

DHN wells

DH-18X

- Well Design

  - Casing program

    - Casing program for Production/injection wells

    - Casing program for Exploration wells

| Hole size (in) | Depth (MD) | Casing Specification |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Size (in) | Grade | Weight (ppf) | Connection Type |  |
| 26 | 500 | 20 | X-56 | 133 | Quick connection |  |
| 16 | 500- 2,200 | 13 3/8 | L80/L80 | 68 | BTC Connection | Casing shoe at Middle H20 |
| 12 ¼ | 2,200 3,530 | 9 5/8 | L80 | 47 | Premium Connection | H-80 |
| 8 1/2 | 3,530 4,315 | 7” | L80 | 29 | Premium Connection | 7” Liner (H200) |

DH-18X

| Hole Size (in) | Depth (mMD) | Casing Specification |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Size (in) | Grade | Weight (ppf) | Connection Type |  |
| 26 | +/- 500 | 20 | X-56 | 133/169 | Quick Connection | Conductor 169ppf from seabed to surface |
| 16 | +/-2,200 | 13-3/8 | L80/N80 | 68 | BTC Connection | Casing shoe at middle of H20 |
| 12 1/4 | +/-3,490 | 9-5/8 | L80 | 47 | Premium Connection | Casing shoe at top of H80 |
| 8 1/2 | +/-4,355 | 7 | L80 | 29 | Premium Connection | Well TD |

    - Casing program for Sidetrack well

Tie-back 20” Conductor & 13-3/8” Casing. Cut 9-5/8” casing & Sidetrack 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner.

    - Casing program for tie back wells

- Drilling Fluid Program

DH-18X

| Depth (mMD) | Hole Size (in) | Mud Type | Mud Weight (ppg) | FV sec | PV cps | YP lb/100ft2 | API FL (cc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 – 500 | 26 | SW/Hi-vis Sweep | 8.8 – 9.2 | >80 | ALAP | >35 |  |
| 500-2,200 | 16 | SW/Hivis –Sweep/ KCL/Gel/Polymer | 8.8 – 9.7 | 45-65 | ALAP | 20-35 | <8 |
| 2,200-3,490 | 12-1/4 | SBM/OBM | 9.5-10.5 | 45 - 70 | ALAP | 25 - 40 | <3 |
| 3,490-4,355 | 8-1/2 | SBM/OBM | 9.5 – 11 | 45 - 70 | ALAP | 25 - 40 | <3 |
| DST |  | CaCl2/NaCl | 10-11 |  |  |  |  |

- Cementing Program

DH-18X

| Hole size - Casing size | Setting depth | Cement interval |  | Slurry |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
|  | mMD/ | Top (m) | Bottom (m) | Density (ppg) | Recipe |  |
| 26’’x20” | +/-498 | Seabed | Shoe | 15.8 | Single G cement | 150% excess in OH or lead up to seabed. |
| 16’’ - 13-3/8’’ | +/-2,195 | 150-200 inside previous shoe | Top tail | 12.8 | Lead G cement | Excess: 50% in OH or 15% Carbide bomb |
|  |  | +/- 250 above shoe | Shoe | 15.8 | Tail G cement |  |
| 12-1/4’’ - 9-5/8’’ | +/-3,485 | 150-200 inside previous shoe | Top tail | 12.8 | Lead Blended/LW cement | Excess: 25% in OH or 10% Caliper. |
|  |  | +/- 250 above shoe | Shoe | 14.5 | Tail Blended/LW cement |  |
| 8 ½” – 7” liner | +/-4,352 | Top of Liner | Liner shoe | 14.5 | Single Blended/LW cement | Excess: 25% in OH or 10% Caliper. |

- Bit Program

DH-18X

| Hole size (in) | Bit Type/IADC | Nozzle /32” | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | From (m) | To (m) | Interval (m) | WOB (kips) | RPM | GPM |
| 26 | Mill tooth (1.1.5) | 3x18, 1x22, 3x16 | 110 | 500 | 390 | 5-15 | 80- 100 | 500-950 |
| 16 | PDC (M423) | 9x12 | 500 | 2,200 | ± 1,700 | 15-35 | 80-150 | 950-1050 |
|  | TCI (1.1.5) |  |  |  |  |  |  |  |
| 12-1/4 | PDC (M223/M323) | 7x14 | 2,200 | 3,490 | ± 1,290 | 15-25 | 60-150 | 640-1050 |
|  | TCI (519/215) |  |  |  |  |  |  |  |
| 8-1/2 DHN-1P | PDC (M223/M323) | 5x14/ 6x13 | 3,490 | 4,355 | ± 865 | 10-30 | 80-150 | 400-600 |

- BHA Program

DH-18X

| Hole size (in) | Interval (mMD) | BHA Program |
| --- | --- | --- |
| 26 | 0 – 500 | 26” Bit + 9-1/2’’ Mud Motor + 9-1/2’’ Stab + 9-1/2’’ MWD (GWD) + 9-1/2” DC + X/O + 8-1/4’’ DC + X/O + 8” Jar + X/O + 15x5-1/2” HWDP |
| 16 | 500 – 2,200 | 16” Bit + 9-1/2” Mud Motor / RSS + 16” Stab + 9-1/2’’ MWD + 9-1/2’’ LWD + X/O + 9-1/2’’ MWD + X/O + 6x8-1/4” DC + 8” Jar + 2x8-1/4” DC + X-Over + 15x5-1/2” HWDP |
| 12-1/4 | 2,200 – 3,490 | 12-1/4” Bit + RSS + 12-1/8” Stab +9-1/2’’ MWD + 9-1/2’’ LWD (Full Option) + FAS/TES TRAK + X/O + 12-1/8’’ Stab + 6x8-1/4” DC + 8” Jar + 2x8-1/4” DC + X/O + 15x5-1/2” HWDP |
| 8-1/2 | 3,490 – 4,355 | 8-1/2” Bit + RSS + 8-3/8” Stab + 7’’ MWD + 7’’ LWD (Full Option) + FAS/TES TRAK + X/O + Agitator + X/O + Shock Sub + 3x6-1/2’’ DC + X/O + 12x5-1/2” HWDP + X/O + 6-1/2” Jar + X/O + 5x5-1/2” HWDP |

- Drilling and Completion Schedule

For 1st Period: Tentatively start in Oct 2026 & Complete in Apr 2027.

For 2nd Period: Tentatively start in July 2027 & Complete in Dec 2027.

In the event that the CLIENT requires the Provision of Mud Logging Services on 10th August 2026, the CONTRACTOR is kindly requested to confirm the service availability at the Mobilization Site at that time

II	SPECIAL PROVISIONS

II.1	Work Site(s)

The Work Site(s) shall be the Drilling Rig(s) at the well location designated by CLIENT.

II.2		Mobilization Date

CLIENT shall give CONTRACTOR at least seven (7) days advance notice for the initial mobilization for CONTRACTOR designated “Call-out” Equipment and CONTRACTOR designated “Call-out” Personnel.  The subsequent mobilization throughout the duration of CONTRACT will be on twenty-four (24) hours on a "call-out" basis.

II.3		CONTRACTOR Contract Administrator

CONTRACTOR hereby appoints, as CONTRACTOR’s Contract Administrator <<To be provided and specified by CONTRACTOR>> or his appointee designated by him in writing.

II.4	CLIENT Contract Administrator

CLIENT hereby appoints, as CLIENT Contract Administrator, the Director or his appointee designated by him in writing.

II.5	CLIENT Representative(s)

CLIENT hereby designates the Director, Drilling Manager, Drilling Superintendent, Senior Drilling Engineers and Drilling Supervisors as CLIENT Representatives.

II.6	Mobilization Site for CONTRACTOR Equipment

The Mobilization Site for CONTRACTOR Equipment shall be the CLIENT’s Warehouse at PTSC Supply Base, Vungtau, S.R. Vietnam or any other mutually agreed location.

II.7		Demobilization Site for CONTRACTOR Equipment

The Demobilization Site for CONTRACTOR Equipment shall be at CLIENT Warehouse at PTSC Supply Base, Vungtau, S.R. Vietnam or any other mutually agreed location.

II.8	Mustering Point for CONTRACTOR Personnel

Southern Vietnam Helicopter Company's Helibase in Vung Tau or Ho Chi Minh City (if required by CLIENT) or PTSC Supply Base, Vungtau for all CONTRACTOR’s Personnel.

II.9	CLIENT Warehouse and Base Facility

PTSC Supply Base, Vungtau, S.R. Vietnam.

II.10	CONTRACTOR Warehouse and Base Facility

<<To be provided and specified by CONTRACTOR>>

II.11	CLIENT Operations Office

15th Floor, Victory Tower, 12 Tan Trao Street, Tan My Ward, Ho Chi Minh City, S.R. Vietnam.

II.12	CONTRACTOR Operations Office

<<To be provided and specified by CONTRACTOR>>

II.13	Commencement Date of CONTRACT

The time and date when CONTRACTOR mobilize the first CONTRACTOR Equipment to the Mobilization Site or the first Personnel at the Mustering Point.

II.14	Commencement of Rates

The applicable rates as defined in EXHIBIT IV hereof shall commence on the Commencement Date of CONTRACT. No payable rates shall be payable throughout mobilization and demobilization of CONTRACTOR Equipment or CONTRACTOR Personnel.

The applicable rates as defined in EXHIBIT IV hereof shall commence upon physical mobilization.

II.15	Point of Origin

CONTRACTOR Point of Origin is at Vung Tau or other location designated by CLIENT for CONTRACTOR's Personnel that are to be furnished under this CONTRACT.

II.16	Delivery Schedule

Delivery schedule for Contractor’s equipment and personnel to be available in Mob/Demobilization Site under DAP Vung Tau base Incoterms 2020 upon receipt of Client’s EQUIPMENT/ MATERIALS/ SERVICES REQUISITION.

The lead time for delivery of CONTRACTOR’s equipment and materials to the Mobilization Site shall be up to 15 days after receipt of Materials Request (MR).

CONTRACTOR’s personnel shall be mobilized to the Mobilization Site as below:

a) Within 48 hours after receipt of first MR; and

b) Within 24 hours after receipt of subsequent MR(s) or any emergency case.

II.17	Request for WORKS

A request for Provision of Remotely Operated Vehicle (ROV) Equipment and Services for WHP-DHN wells, Block 05.1a, Offshore Vietnam shall be either via telex, facsimile, telephone or verbal instructions to be followed by telex or facsimile from CLIENT’s Contract Administrator.

II.18	Duration of CONTRACT

The duration of the CONTRACT shall be as per ARTICLE 3.

II.19	“Call-Out” of CONTRACTOR Personnel

As and when directed by CLIENT on “Call-out” basis.

II.20	“Call-Out” of CONTRACTOR Equipment

CONTRACTOR shall make available CONTRACTOR Equipment at CONTRACTOR Warehouse in Vung Tau, Vietnam for delivery to the Work Site as requested by CLIENT on “Call-out” basis.

II.21	Completion of WORKS

Completion of WORKS shall be achieved when the operations described in EXHIBIT I-2.0 on the last well designated by CLIENT has been completed and CONTRACTOR has been notified in writing by CLIENT.

II.22	Demobilization of CONTRACTOR Equipment

Upon completion of WORKS on the last well designated by CLIENT, CONTRACTOR shall make ready all Equipment for offloading onto the supply vessel(s). CONTRACTOR shall be responsible for the removal of CONTRACTOR's Equipment from the designated Demobilization Site. Rental shall cease when CONTRACTOR Equipment reaches the Demobilization Site.

II.23	Release of CONTRACTOR Equipment

CLIENT shall have the right to release any of the CONTRACTOR Equipment for any reason whatsoever within twenty-four (24) hours.  CLIENT shall endeavor to notify CONTRACTOR of its intent to release any of CONTRACTOR Equipment as soon as such decision is known.  However, CLIENT shall continue to pay the applicable rental rates until the CONTRACTOR Equipment has been delivered and reached to the Demobilization Site.  CONTRACTOR shall promptly prepare for demobilization whatever portion of CONTRACTOR Equipment that is to be released and shall assist CLIENT in the loading-out of the said equipment.  CONTRACTOR shall be on hand at the Demobilization Site to take delivery of such equipment.

II.24	Re-engagement of CONTRACTOR Equipment

CLIENT shall have the option to re-engage any released CONTRACTOR’s Equipment.

II.25	Release of CONTRACTOR Personnel

CLIENT shall give CONTRACTOR at least five (5) days advance notice of release of any designated personnel.  However, subject to terms and conditions of main contact body hereto, CLIENT shall have the right to release without prior notice any of CONTRACTOR Personnel, who, in CLIENT’s opinion, are or could be detrimental to the performance of the WORKS or interest of CLIENT.

II.26	CONTRACT Closure

CONTRACTOR shall submit to CLIENT a “CONTRACT CLOSURE LETTER” and “CONTRACT CLOSURE CERTIFICATE” as per format shown in APPENDIX X-6 and APPENDIX X-7 respectively.

II.27	Health, Safety and Environment Requirement

CONTRACTOR shall provide necessary tools, equipment and written procedure to make sure all hazards listed in APPENDIX VI-5 are addressed.

CONTRACTOR is required to develop the action plan (or written procedure) for each hazard listed in APPENDIX VI-5 (where and when applicable) prior to the commencement of WORKS.

II.28	Changes

II.28.1	Should CLIENT desire any change to the CONTRACTOR’s Equipment, it shall advise CONTRACTOR of said request.  CONTRACTOR shall, upon request from CLIENT, provide CLIENT within fourteen (14) days a CHANGE PROPOSAL defining the terms and conditions shall include but not limited to price, method of payment, earliest commencement date and any other information deemed necessary.

II.28.2		When and if CLIENT approves the CHANGE PROPOSAL, CLIENT will issue to CONTRACTOR a written CHANGE ORDER in duplicate originals in the form shown in APPENDIX IX-2 herein, CONTRACTOR shall sign both duplicate originals of the CHANGE ORDER to indicate its receipt, understanding and acceptance of it. Execution by CLIENT, one duplicate original will be returned to CONTRACTOR.

II.28.3		Should the Parties agree to the terms and conditions, including but not limited the Price, and Method of payment, the change shall be made

III.	REQUEST FOR SERVICES

- The Works to be performed by the Client shall, as and when requested by the Client shall be the provision of Mudlogging Equipment and Services for the duration of the Client’s Drilling Operations.

- The Works shall include all associated services onshore and the provision of technical support and Contractor Group personnel as required prior to mobilization of Contractor Group Equipment to assist the Client in the operational planning.

- In the performance of the Works, the Contractor shall provide all Contractor Group Equipment, Materials, Contractor Group personnel and all other items necessary (including back-up equipment) for the provision of the Works.

- The Contractor shall comply with the specification for performance of the Works included in this Schedule as well as the general items specified by Client.

- The Contractor shall be responsible for the safe and effective performance of the Works and shall investigate and report on all incidents and operational failures in a timely manner in accordance with Client requirements.

IV.	SCOPE OF WORKS

The CONTRACTOR is required to provide the following to CLIENT.

- To provide one (01) skid mounted Mud Logging Units.

- To provide the field service crew comprising Predict pressure engineer, Data Enginners / Mud Logging Crew Supervisors, Mudloggers and Sample Catchers who are fully experienced, trained and qualified as required by Company. The number of contractor personnel based at the WORKSITE may be specified at any time by company personnel.

- To perform mud logging and associated services including but not limited to formation sampling and analyzing, handling and analyzing of cores and side wall samples, (guns and/or rotary coring tool) gas detection analysis and evaluation, drilling and mud parameter monitoring, formation pressure evaluation and preparation of master log, plots and reports with all items of equipment and ancillary and optional equipment as described in this Scope of Works in accordance with the well specifications.

- Provision of general assistance to and co-operation with CLIENT and other contractors performing other services relating to the WORKS, as and when required.

- To provide full set of Sample packing material (Wooden boxes, Big cardboard Boxes, Small cardboard Boxes, Cloth bags, Plastic bags, Samples tray, Envelopes, and Bottle samples).

- To attend safety and planning meetings with CLIENT’s and other contractors representatives on safety aspects, work planning and procedures to be carried out prior to any drilling sequence and otherwise execute all WORKS in connection with this Scope of Works.

The WORKS is defined and described by, but not necessarily limited to any or all of the following:

1.	Sample Collection and Package

CONTRACTOR should give special attention, so that the samples are collected in sufficient quantity and packaged in the proper order as per CLIENT’s requirement. Items required for collection of samples and packaging are specified below:

| ITEM | PURPOSE |
| --- | --- |
| Plastic bags, Cloth bags | Unwashed and Washed samples |
| Big and small Boxes, Cardboard | Storage, Shipment of dried samples |
| Samples tray | Storage, Shipment of description samples |
| Bottle sample | Storage, Shipment of Mud samples |
| Boxes, Wooden | Storage, Shipment of unwashed & mud samples |
| Envelopes, paper with fold over closure | Packing of individual washed and dried samples |
| Sieves | Standard (ASTM Sieve No. 25, 40, 80, 170, 230 and 270) |

As soon as samples are returned to surface, samples shall be collected and examined each and every 5m or 10m as mentioned in well proposal. At each sample depth, two (2) sets of unwashed cuttings shall be taken, and two (2) sets of dried cuttings samples shall be prepared by CONTRACTOR.

CONTRACTOR will ensure accurate collection, washing, drying, examination and bagging of cuttings, sidewall core, core, MDT and mud samples as laid out in this Scope of Work and as directed by the well site geologist.

CONTRACTOR will keep a complete record of all samples taken, samples interval, changes of sample interval, empty samples, additional samples such as bit samples, sidewall core samples, etc. All samples movements from the rig will be accompanied by a transmittal, copies of which will be maintained by CONTRACTOR and submitted to the CLIENT at the end of the well.

- Unwashed and wet samples

Two (2) sets (A, B) of unwashed wet samples (not less than 500 gram) will be collected at the specified intervals and bagged in sealable barrier foil or polythene lined cloth bags.

All samples are to be clearly, legibly and indelibly labeled with Operator, Well Name, Depth Interval and Set, using a waterproof marker pen. When preparing the sample bags, Contractor shall ensure that the labeling is permanent. Unwashed cuttings samples should be packed in strong, labeled boxes for shipping to the Company distribution center, address to be provided prior to spudding the first well. The sets of wet cuttings shall be dispatched as soon as possible after reaching total depth for a particular hole section or earlier if logistics permit.

Contractor shall ensure that the collection of cuttings is made after correction for lag time, so that the sample depth corresponds to the drilled depth of that sample. Lag times should be established initially by calculation and checked by a carbide test every 200 meters or as requested by the Company Representative.

Samples should be caught by placing a wooden board across the front of each shaker. Take the sample by cutting vertically down through the pile of cuttings. If different screens are used on a twin shaker use two boards and make the sample up from both piles. Normally the proportion should be approximately equal. The board must always be hosed down after each sample is collected. It is important to emphasize to the shaker hand that the cuttings board should not be hosed off between visits by the mud logger.

During fast drilling, the board will overload quickly, so the sample becomes non-representative. It is better in this case to bulk the sample using two separate collections, one at half lag time, and the other at full lag time. During slow drilling, the same procedure may need to be repeated several times in order to get enough quantity.

If a sample is missed, seal the relevant bag empty and place with the other samples. Notify the well site geologist who will consider whether to change the sampling interval and note the depth interval missed in the morning report.

Whenever the desander or desilter is in use, collect samples from the cyclones and describe them on the work sheet. Depending on the size of shaker screens in use, combine this information in the overall sample description.

If a trip results in a sample being missed, always catch a “TD” sample and mark the bag with the back-calculated depth.

The set of sample sieves must be complete and in good condition. Sieves normally used are 2mm, 0.250 mm and 0.063 mm. The raw wet sample should be put into top (coarsest) sieve and gently washed through to the lower, finer sieves with tap. Care should be taken not to destroy any sensitive mudstone cuttings. If in doubt, reduce the amount of washing.

The residue in the 5 mm sieve usually represents caving. The amount, and any changes should be noted, and the information communicated to the well site geologist. The contents of 0.250 mm sieve are retained for microscopic and fluorescence analysis and dried sample. Periodically examine the contents some of the other sieves for comparison.

During washing, look out for and record any oil skim on the water after passing over the samples.

- Washed and dried samples

Two (2) sets (A, B) of washed and dried samples (approximately 100 gram) are collected from the washed portion of cuttings in the 0.250 mm sieve, oven-dried and bagged in strong envelopes and packed in cardboard boxes. Only fully dried material should be packed. The oven’s temperature shall not exceed 90°C.

All samples are to be clearly, legibly and indelibly labeled with Operator, Well Name, Depth Interval and Set. A cover note in a plastic envelope shall be stapled to the outside of the box. As soon as the carton is full it shall be handed over to the Company wellsite representative for dispatch to the office on the first available transport.

- Empty samples

If for some reason no samples are recovered, the mud logger will report this fact in daily morning report. The mud logger will make sure that empty bags, labeled in depth increments of the current sample interval are included with the routine unwashed and washed and dried samples.

- Cavings

One set (M) of cavings sample will be collected at every 5m interval (where present) and photographed according to PVEP POC standards (to be provided). The samples will then be bagged in sealable barrier foil or polythene lined cloth bags. If samples are larger than the bags, then an alternative collection method should be available.

- Mud samples

One set (P) of mud sample will be taken from the flowline every 100m and collected in an airtight plastic can with a suitable amount of bacteriacide added. Mud property determination is necessary to properly evaluate wireline log responses and drilling characteristics. Samples will also be taken:

When an oil skim is observed at the shakers or in the mud pits.

While drilling hydrocarbon bearing sections.

When there is a significant change in fluid characteristics.

Prior to coring and after coring.

Prior to logging.

An organic contaminant, such as diesel oil is added to the mud.

A mud filtrate tracer is running.

At spud, samples of all mud materials intended for used in the well should be collected from the mud engineer. These will be kept in the Mud Logging Unit for the duration of operations. Their fluorescence under UV light should be noted for reference when examining cuttings. Samples of all mud additives will be collected with the date, time and depth at which they were added to the mud system. They will be sent to Company’s Vung Tau warehouse along with samples at T.D.

###### Cuttings sampling should be taken as the table below:

| Depth (mMD) | Frequence of sample |
| --- | --- |
| From 500 mMD to TD well | Every 5m |

At any time, while drilling in a potentially significant show, samples will be caught and examined every meter or as specified by PVEP POC Representative.

Sample Bagging

All samples must be properly labeled, including Operator, Well Name, Depth Interval, Type Sample and Destination.  A summary of these requirements is tabulated as Table bellow:

| Type | Number of sets | Frequency | Weight | Container | Partners |
| --- | --- | --- | --- | --- | --- |
| Unwashed & Wet cutting sample | 2 | Every 5m take 01 sample | 500 gram /sample | Polythene or cloth bag | PVN & PVEP-POC |
| Washed & Dried cutting sample | 2 |  | 200 gram /sample | Paper/plastic envelope | PVN & PVEP-POC |
| Mud Sample | 1 | As above/ Request by WSG | 1 litter | Plastic can | PVN & PVEP-POC |

CONTRACTOR will label all samples with proper destination as above and send to PVEP-POC Vung Tau supply base. PVEP POC Vung Tau supply base should distribute the samples as labeled with proper documentation.

2.	Geological Surveillance Work

Geological surveillance work is required to be carried out round the clock at drilling site in Mud Logging Unit as detailed below:

2.1 	Examination / Analysis:

During Sample Analysis/Examination, MLU personnel must be concerned about the followings:

          - Perform microscopic examination of samples to identify rock types, percentages and visual characteristics using comparator chart to determine grain size, sorting, roundness and standard classification system. Additionally, the lithology description should include the followings:

        - Color, Fabric / Texture, hardness, grain size etc.

        - Mineral identification (calcite, clay, silica, metallic minerals etc.)

        - Visual Porosity/Permeability estimation

          - On lithology work sheet, to write sample description for each new lithology, using standard terms and order. If necessary, to perform selective Calcimetry study, Shale Density and any other analysis required by COMPANY.

          - The foreign materials should not be included in lithology description. Foreign material includes caving from the hole, and where identifiable, mud additives, loss circulation material, cement and pipe dope etc. If any foreign materials are observed, then it should be noted on the Master Log. MLU personnel have to take care of the following hydrocarbon detection method:

          - Direct/ Cut UV fluorescence

          - Residual

        - Intensity of any petroleum odor.

        - Staining and Bleeding.

        - Gas composition

2.2 	Gas analysis: Detection of gas from C1 through C5 along with volume percentage and gas identification (formation, connection, pump off etc.)

3.	Detection of Down-hole Problems / Anomalies, etc.

- High / Abnormal Pressure Prediction: Analysis / Computation of relevant data for detection of high-pressure zones based on Dc Exponent, Shale Density, Shale Factor and detection of H2S, CO2 etc. with accuracy up to 10ppm and 5% respectively.

- Immediate detection of kick and monitoring of pipe pressure.

- Detection of mechanical failure of bit, drill string and pumps.

- Promptly inform Rig floor, Company Man & Wellsite Geologist above down-hole problems.

4.	Determination of Geological / Drilling Parameters

- Determination of Shale Density.

- Determination of kill mud weight, volume, etc.

- Real time plots of continuous recording of gas and FID based analysis of gas (C1-C5).

- Determination of volume of mud loss in case of circulation loss.

- Determination of pressure drops in mud circuit, jetting velocity, bottom dynamic pressure and Equivalent Circulation Density.

- Determination of Gas Quality Control.

5.	Plotting and Preparation of Data

5.1	Mud Log: To use an industry standard mud log format, generate and maintain current mud log in the computer and prepare their plots as requested by COMPANY geologists in 1:500 scale including the followings:

- The mud log should display lithological information in two ways, they are

  - Percentage, in which the rocks are described as a percentage of an aggregate sample over a specified interval.

  - Interpretative, in which recovered rocks are correlated with drill rate and gas. An abbreviated lithology description is to be given in the remarks column.

- Plotting should be - both the Total Gas curve (%) and the Component Gas curves (ppm) on logarithm log scales. Plot on gas track should include formation gas, connection gas, trip gas and carbide lags etc.

- Presentation of mud weight in/out and mud properties should be in the first track on every 100m drilled. Mud weights need to be recorded as a comment whenever a mud weight change is made.

- To record hole problems (Ex. Mud losses/gain, drill pipe sticking etc.)

- To record all casing and logging points.

- To record all bit trips, bit number, bit size and type.

- Mud additive information, LCM etc. on the log (as Remarks).

- To plot the bit penetration rate (ROP) as meter/hour. Gamma ray (from MWD/ Wireline) plot should be increasing towards right side.

5.2	Gas Ratio Log: The log should be prepared in 1:500 scale with a summary of interpretation displaying the following ratios:

- Wetness Ratio (Wh) 	= (C2+C3+C4+C5) / (Cl+C2+C3+C4+C5)

- Balance Ratio 	(Bh) 	= (C1+C2) / (C3+C4+C5)

- Character Ratio 	(Ch) 	= (C4+C5) / C3

5.3	Lag times

- Carbide lag check is to be recorded whenever required. Record resultant gas show on chromatograph mud log as response per gram of carbide.

- Carbide lag check should remark on the Master Log include hole size calculated and time in minutes.

5.4	Other Requirement

- Geo-pressure log: To prepare in 1:1000.

- TVD plot: In case of a deviated well, deviated section of the hole to be plotted after computation of the data.

- Well profile and drilling progress chart: To be plotted on regular basis / daily.

- Plotted data to be routinely (as advised) handed over/forwarded to CLIENT representative.

- Daily progress reports (including hydraulic report, mud loss report, etc.) to be submitted to CLIENT’s Presentative(s) every morning or as advised by the operation geologist.

- An updated PDF file of the Mud log, Pressure log, Drilling log, Gas log (in depth) and daily time log (drilling & gas data) are to be given to COMPANY’s well-site geologist as well as send to CLIENT’s operational office daily. Print out of Mud logs should be furnished as & when required by the Geologist at well-site and operational office.

- Drilling log: The log should be prepared with a summary of important drilling events in 1:500 scales.

6.	Digital data storage

To store acquired, measured and observed data digitally Bidder has to provide the digital data on compact disk in LAS or ASCII format to CLIENT, immediately after completion of the well. All the time & depth based data to be stored and provided to the CLIENT at the completion of well in USB Thumb drive(s).

7.	Reporting and Data Requirements

CONTRACTOR will be required to provide CLIENT with regular reports at the well site on a daily basis together with job ticket and end of well reports.

- Daily Reporting

A daily report including all data required by CLIENT, together with an updated Master log and any other requirements and comments relating to significant events will be prepared each morning for the previous twenty four hour period and submit to CLIENT’s well site geologist by 06:00 for the daily geological/drilling report.

The exact log format will be determined separately. At a minimum, logged and recorded information will include depth, ROP, mud and bit information, significant engineering items, cutting and interpreted lithology, cutting shows, total gas, component gas through nC5 and selected gas ratios.

CONTRACTOR will prepare an evaluation report prior to each mud logging crew change including Equipment performance, mud logging crew performance and any other pertinent items of interest or concern. A copy will retain at the wellsite and the original forwarded to the CLIENT.

- Job Ticket Reporting

CONTRACTOR will be required to furnish a monthly job ticket completed by CONTRACTOR. Tickets must include as a minimum, the data listed below:

- Location identification giving times and dates of services, well name, drilling rig name, and permit.

- Names and job titles of all CONTRACTOR Personnel involved and dates and time of arrival and departure from the Drilling Rig during that job if applicable.

- Report of any incidents or malfunctions of CONTRACTOR’S Equipment and/or job and reason thereof, outlining the problem on the job ticket and with an attached detailed report. Also any such incident or malfunction should be immediately reported to the CLIENT’s well site representative.

- Itemised costing breakdown of all items relating to the job and any other items deemed relevant for invoicing purposes.

- Job ticket to be signed by CLIENT’s drilling rig supervisor who will forward to the Company’s Ho Chi Minh office for attention to CLIENT Representative.

- Mudlogging End of Well Report

Four (04) original Mudlogging End of Well Report must be submitted to CLIENT in hard copies and all the data in USB within 15 (fifteen) days from completion of rig down of the Unit.

Mudlogging End of Well Report consisting of a compilation of all data collected including but not limited to bit data, mud data and deviation survey data as well as geological data; and a summary outline of operational aspects which may have affected or influenced the mud logging process (e.g. circulating bottoms-up gas, maximum percentage recorded and duration of gas peak, occurrence of torque, overpull, etc.); a knowledge of which is essential to arrive at a correct evaluation of the data collected on site. The report should also contain a summary interpretation of the lithology and hydrocarbon indications data collected.

USB of final well report based on ASCII format are required. End of well reports will include but not limited to the following: Depth versus day, well diary, bit data, mud data, casing data, drilling summary, “D”-exponent plot, gas data, temperature profile, drilling parameters, etc.

## 8.	Accessories

The CONTRACTOR shall make available the following accessories in the Mud Logging Unit:

- An explosion proof purged LCD monitor with real time display should be installed at the Rig floor along with provision for an intrinsically safe audio alarm.

- A Suitable LCD monitor to display various real time sensor signals, parameters and to represent graphical display of parameters with time-to-time comments in the mud logging unit.

- Necessary hardware & software to back up all recorded time-based mud logging data for the entire well operations.

- Online repeater LCD monitor (minimum 19”) to be installed in each of Company man’s office, OIM’s office, Tool pusher’s office, Wellsite Geologist’s office, Mud Lab, MWD unit, Wireline unit and Third party’s office.

- Back-up facility of 24 hours online graphical screen saving with comments.

- 02 Ditch Magnet of 01-meter length.

- The number of pit sensors should be fitted with the number of Rig’s pit.

- 03 Pressure sensor at Well head, SPP and cement unit.

- All other supplies as required to complete the SERVICES, including, but not limited to bactericide for samples intended for geochemistry and paleontological analysis, aluminums foil, plastic cling wrap, wax, packing tape, indelible maker pens, etc.

- Minimum 02 each standard CO2, H2S sensor in Unit and flow line.

## 9.	Back up equipment and materials

CONTRACTOR is responsible for supplying all back-up equipment and materials in order to provide smooth and uninterrupted mud logging services to CLIENT. A minimum of one set of back up sensors, gas equipment and paper etc. must be available at MLU.

## 10.	Other services

CONTRACTOR Personnel will be responsible for supervision and co-ordination for the above and also for the analysis and report of the followings:

10.1 	Labeling cuttings and fluid samples:

- To mark directly on can or bag in large legible letters in English. Do not use attachable labels. Use a waterproof marking pen on bags.

- Labels should state the well name, sample depth in meters and sample type (i.e. unwashed and wet cuttings, SWC or whole core).

10.2 	Daily summary of vital drilling parameters: Monitor of pit level and hole volumes. Provide summary detailing estimation of mud losses and gains.

10.3	Conventional and Side-Wall Core samples: MLU personnel need to assist the well-site geologist or other CLIENT representatives during coring operations. Mud logger’s duty during coring job is as under:

- Collect circulated samples during drilling operation.

- Monitor drilling parameters for signs of problems down-hole.

- When core is brought to surface, the Mud logger has to assist in the laying out of the core on the drill floor, assist during initial analysis and packaging prior to transportation.

- Wire-line operator shall provide packaging of side-wall cores. Mud logger shall provide assistance in the description and handling of side-wall cores as directed by the CLIENT’s well site engineer and geologist.

11.	Real-time Services at Office:

Remote display LCD/LED screen capable of displaying graphic and alpha numerical real time data from Mud Logging Units shall be placed in the CLIENT’s Geology & Drilling Department. The real time monitoring system should also include necessary software for manual playback/ scroll back for viewing earlier events. The Real Time Monitors system should include

- Remote, secure, real-time monitoring of time and depth based data from any global location.

- Data viewable through any web browser / web enabled device that include PC, laptop, smart phone and tablets (apps based).

- Support and visualization for mud logging & drilling data.

- Number of access licenses – To be provided and specified by CONTRACTOR.

- Optional facility for streaming in of third party data like MWD, LWD, or any other data stream in WITS standard, from the rig site.

CLENT will provide necessary communication from drilling location to CLIENT’s office at HO CHI MINH City. However, BIDDER has to ascertain the integrity of data in case of intermittent failure of communication link (i.e. data sync during communication failure).

12.	Confidentiality of Information

All documents, data and other information which have not been made public and which come into CONTRACTOR’s possession or to his attention in the course of rendition of the SERVICES shall be CLIENT’s sole property and shall not be used in whole or in part by CONTRACTOR other than as necessary hereunder or released or disclosed in whole or in part by CONTRACTOR to anyone not employed by CLIENT without CLIENT’s written consent (except where required by court order or subpoena, copies of which shall be timely furnished to CLIENT prior to release or disclosure). Without CLIENT’s prior written consent, neither CONTRACTOR nor any person acting for or on behalf of CONTRACTOR will confirm or deny any statement to any third parties regarding any information provided to CONTRACTOR hereunder or disclose to any third party that such information has been provided to CONTRACTOR. All reports, documents, data, presentations and other material prepared by CONTRACTOR pursuant to this Agreement shall be CLIENT’s sole property in which CONTRACTOR shall have no proprietary rights (including, but not limited to, copyright). Upon termination of this Agreement, CONTRACTOR will promptly deliver to CLIENT all documents, data and other information and all copies of same, and any analyses, compilations, studies or other data prepared hereunder in CONTRACTOR’s possession or control. This confidentiality obligation, which shall be strictly construed, shall survive the termination of this Agreement. CONTRACTOR will require all of its employees and Sub-CONTRACTORs to agree to and abide by terms identical to this Item.

13.	Training of CLIENT personnel

CLIENT seeks opportunity for the training of its personnel in the well testing services and related operations. Accessibility to CONTRACTOR’s training facilities within or outside Vietnam will be a consideration in the bid evaluation exercise. All expenses so incurred by CLIENT personnel would be for account of CLIENT.

14.		Technology Transfer Incentive (TTI)

CONTRACTOR is encouraged to submit to CLIENT a TECHNOLOGY TRANSFER INCENTIVE (TTI) plan, it also will be a consideration in the bid evaluation exercise. This shall include but not limited to Manufacturing site visits, technology training for CLIENT’s personnel, technical workshops.

V.	CONTRACTOR Work Performance Evaluation

CLIENT shall continuously evaluate the work performance of CONTRACTOR pursuant to the requirement of the CONTRACT throughout the term of the CONTRACT.