EXHIBIT I

SPECIAL PROVISION AND SCOPE OF WORKS

| TABLE OF CONTENTS |  |  |
| --- | --- | --- |
| SECTION | TITLE |  |
| I-1.0 | GENERAL INFORMATION |  |
| I-2.0 | SPECIAL PROVISIONS |  |
| I-3.0 | SCOPE OF WORKS |  |
| I-4.0 | WORK ORDER REQUEST |  |
| I-5.0 | CONTRACTOR’S WORK PERFORMANCE EVALUATION |  |

EXHIBIT I

SPECIAL PROVISIONS AND SCOPE OF WORKS

I-1.0	GENERAL INFORMATION

I-1.1	EXPLORATION WELLS:

- AREA OF OPERATIONS:		Block X (Open water), Block 05.1a; Offshore Vietnam

- WELL NAME	:			DH-18X

- WELL TYPE:				Exploration/DST

- START TIME:			Tentative in August 2026 for DH-18X

- ESTIMATED TIME			122 days

- WATER DEPTH			+/-109m

- RIG TYPE				Jack up 400ft WD

I-1.2	DEVELOPMENT WELLS:

- AREA OF OPERATIONS:		WHP-DHN, Block 05.1a; Offshore Vietnam

- WELL NAME	:			DHN-4X, DHN-2X, DHN-1X, DHN-1P, DHN-2P,

DHN-3P, DHN-4PI, DHN-5PI, DHN-6PI

- WELL TYPE:				Tie-back/Production + Production +

Production/Injection wells

- START TIME:			Tentative in October 2026

- ESTIMATED TIME			350 days

- WATER DEPTH			+/-109m

- RIG TYPE				Jack up 400ft WD

- Well information:

Total Well at Block X (Open water) location: DH-18X, these wells are exploration type with Targets are at Carbonate and Clastic reservoirs.

Total of 9 Wells at WHP-DHN:  3 tie-back/production, 3 production and 3 injection/production wells with well Targets are at Thong – Mang Cau formation (Carbonate) from H44 – H76.2.

| WHP | Well name / Type |  | Slot | Note |
| --- | --- | --- | --- | --- |
| DHN | DHN-1X | Production | #5 | Block A7 (H44-H95) |
|  | DHN-2X |  | #9 | Block A8 (H44-H80) |
|  | DHN-4X |  | #6 | Block A7.2 (H44-H76.2) |
|  | DHN-1P |  | #1 | Sidetrack from DHN-3X (H44-H76-2) Block A7.2 |
|  | DHN-2P |  | #2 | Block A8 (H46-H76.1.3) |
|  | DHN-3P |  | #3 | Block A7.1 (H50-H76.1) |
|  | Total: 6 |  |  |  |
|  | DHN-4PI | Production/ Injection | #4 | Block A7.2 (H44-H76.2) |
|  | DHN-5PI |  | #7 | Block A8 (H50-H76.1.3) |
|  | DHN-6PI |  | #8 | Block A7.1 (H46-H76.1) |
|  | Total: 3 |  |  |  |

- Well Structure

  - Well Structure for Production/Injection Well at WHP-DHN:

26” hole/20” conductor x 16” hole/13-3/8” casing x 12-1/4” hole/9-5/8” casing

Figure 1: Typical well Structure for Production/Injection

  - Well Structure for Sidetrack Well at WHP-DHN:

Tie-back 20” Conductor, 13-3/8” Casing, 9-5/8” Casing. Cut 9-5/8” casing & Sidetrack 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner

Figure 2: Well Structure (Sidetrack) for DHN-1P

  - Structure for Tie-back Well at WHP-DHN:

- DHN-4X and DHN-2X: Tie-back 20” Conductor, 13-3/8” Casing, 9-5/8” Casing from seabed to surface.

- DHN-1X: Tie-back 20” Conductor, 9-5/8” Casing from seabed to surface. Tie-back 7” from +/-3,257m to surface.

  - Well Structure for Exploration Wells at Block X:

26” hole/20” conductor x 16” hole/13-3/8” casing x 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner.

- Completion Schematic

  - Completion Diagram for Production Well

Figure 3: 3 ½” Tubing & 13Cr Material

  - Production/Injection Well

Figure 4: 4 ½” Tubing & L80 Material

- Well Lay-out and Drilling Schedule

11 Slots: 2 slots (Dual for future wells – Blue color) + 9 slots x 20” Conductor.

Figure 5: Well slot lay-out for WHP-DHN

Figure 6: WHP-DHN

- Basic Geological Well information:

The well is expected normal temperature and normal pore pressure.

  - Lithology:

  - Pressure, Temperature Profiles:

- WELL DESIGN

- Casing Program:

  - Casing Program for Production/Injection Well

20” conductor x 13 3/8” casing x 9 5/8” casing

| Hole Size (in) | Depth (mMD) | Casing Specification |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Size (in) | Grade | Weight (ppf) | Connection Type |  |
| 26 | 400-600 | 20 | X-56 | 133/169 | Quick Connection | Conductor 169ppf from seabed to surface |
| 16 | 2,050-2,600 | 13-3/8 | L80/N80 | 68 | BTC Connection | Casing shoe at top of H30 |
| 12 1/4 | 2,900-4,600 | 9-5/8 | L80 | 47 | Premium Connection | Casing shoe at top of H80 |

Figure 7: Casing Specification for Production/Injection Well

  - Casing Program for Sidetrack Well:

Tie-back 20” Conductor & 13-3/8” Casing. Cut 9-5/8” casing & Sidetrack 12-1/4” hole/9-5/8” casing x 8-1/2” hole/7” liner

| Hole Size (in) | Depth (mMD) | Casing Specification |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Size (in) | Grade | Weight (ppf) | Connection Type |  |
|  | 160 | 20 | X-56 | 169 | Quick Connection | Tie-back |
|  | 160 | 13-3/8 | L80/N80 | 68 | BTC | Tie-back |
| 12-1/4 | 2,360 | 9-5/8 | L80 | 47 | Premium | Casing shoe set at Top of H30 |
| 8-1/2 | 4,250 | 7 | L80 | 29 | Premium | 7” liner |

Figure 8: Casing Specification for Sidetrack Well

  - Casing Program for Tie-back Well:

| Hole Size (in) | Depth (mMD) | Casing Specification |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Size (in) | Grade | Weight (ppf) | Connection Type |  |
|  | 160 | 20 | X-56 | 169 | Quick Connection | Tie-back |
|  | 160 | 13-3/8 | L80/N80 | 68 | BTC | Tie-back |
|  | 160 | 9-5/8 | L80 | 47 | Premium | Tie-back |
|  | 4,250 | 7 | L80 | 29 | Premium | 7” liner (DHN-1X only) |

- Drilling Fluid Program

| Depth (mMD) | Hole Size(in) | Mud Type | Mud Weight (ppg) | FV sec | PV cps | YP lb/100ft2 | API FL (cc) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400 – 600 | 26 | SW/Hi-vis Sweep | 8.8 – 9.2 | >80 | ALAP | >35 |  |
| 2,175-2,588 | 16 | SW/Hivis –Sweep/ KCL/Gel/Polymer | 8.8 – 9.7 | 45-65 | ALAP | 20-35 | <8 |
| 2,356-4,594 | 12-1/4 | SBM/OBM | 9.5-10.5 | 45 - 70 | ALAP | 25 - 40 | <3 |
| 4,250 | 8-1/2 | SBM/OBM (DHN-1P only) | 9.5 – 10.5 | 45 - 70 | ALAP | 25 - 40 | <3 |
| Completion |  | Cacl2/NaCl | 10-11 |  |  |  |  |

Figure 9: Drilling Fluid Program

- Cementing Program

| Hole size - Casing size | Setting depth | Cement interval |  | Slurry |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
|  | mMD/ | Top (m) | Bottom (m) | Density (ppg) | Recipe |  |
| 26’’x20” | 400-600 | Seabed | Shoe | 14.0 | Single G cement | 150% excess in OH or lead up to seabed. |
| 16’’ - 13-3/8’’ | 2,170- 2,580 | 150-200 inside previous shoe | Top tail | 12.8 | Lead G cement | Excess: 50% in OH or 15% Carbide bomb |
|  |  | +/- 250 above shoe | shoe | 15.8 | Tail G cement |  |
| 12-1/4’’ - 9-5/8’’ | 2,350-4,590 | 150-200 inside previous shoe | Top tail | 12.8 | Lead Blended/LW cement | Excess: 25% in OH or 10% Caliper. |
|  |  | +/- 250 above shoe | shoe | 14.5 | Tail Blended/LW cement |  |
| 8 ½” – 7” liner (DHN-1P only) | 4,250 | Top of Liner | Liner shoe | 14.5 | Single Blended/LW cement | Excess: 25% in OH or 10% Caliper. |

Figure 10: Cementing Program

- Bit Program

| Hole size (in) | Bit Type/IADC | Nozzle /32” | Bit Performance |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | From (m) | To (m) | Interval (m) | WOB (kips) | RPM | GPM |
| 26 | Mill tooth (1.1.5) | 3x18, 1x22, 3x16 | 110 | 500 | 390 | 5-15 | 80- 100 | 500-950 |
| 16 | PDC (M423) | 9x12 | 500 | 2,175/ 2,588 | ± 2,000 | 15-35 | 80-150 | 950-1050 |
|  | TCI (1.1.5) |  |  |  |  |  |  |  |
| 12-1/4 | PDC (M223/M323) | 7x14 | 2,175/ 2,588 | 2,955/4,594 | ± 2,000 | 15-25 | 60-150 | 640-1050 |
|  | TCI (519/215) |  |  |  |  |  |  |  |
| 8-1/2 DHN-1P | PDC (M223/M323) | 5x14/ 6x13 | 2,356 | 4,250 | ± 1,900 | 10-30 | 80-150 | 400-600 |
|  | TCI (447/215/519) |  |  |  |  |  |  |  |

Figure 10: Bit Program

- BHA Program

| Hole size (in) | Interval (mMD) | BHA Program |
| --- | --- | --- |
| 26 | 0 – 500 | 26” Bit + 9-1/2’’ Mud Motor + 9-1/2’’ Stab + 9-1/2’’ MWD (GWD) + 9-1/2” DC + X/O + 8-1/4’’ DC + X/O + 8” Jar + X/O + 15x5-1/2” HWDP |
| 16 | 500 – 2,175/2,588 | 16” Bit + 9-1/2” Mud Motor / RSS + 16” Stab + 9-1/2’’ MWD + 9-1/2’’ LWD + X/O + 9-1/2’’ MWD + X/O + 6x8-1/4” DC + 8” Jar + 2x8-1/4” DC + X-Over + 15x5-1/2” HWDP |
| 12-1/4 | 2,175/2,588 – 2,955/4,594 | 12-1/4” Bit + RSS + 12-1/8” Stab +9-1/2’’ MWD + 9-1/2’’ LWD (Full Option) + X/O + 11-3/4’’ Stab + 6x8-1/4” DC + 8” Jar + 2x8-1/4” DC + X/O + 15x5-1/2” HWDP |
| 8-1/2 DHN-1P | 2,365–4,250 | 8-1/2” Bit + RSS + 8-3/8” Stab + 7’’ MWD + 7’’ LWD (Full Option) + X/O + 3x6-1/2’’ DC + X/O + 12x5-1/2” HWDP + X/O + 6-1/2” Jar + X/O + 5x5-1/2” HWDP |

Figure 12: BHA Program for Development Campaign

- Time plan

6.1	Time plan for Tie-back Well:

6.2	Time plan for Sidetrack Well:

6.3	Time plan for Production/Injection Well:

Drilling & Completion Schedule: Tentatively start in Aug 2026 & to be completed in Dec 2027

I-2.0	SPECIAL PROVISIONS

I-2.1	Work Site(s)

The Work Site(s) shall be the Drilling Rig(s) at the well location designated by CLIENT.

I-2.2		Mobilization Date

CLIENT shall give CONTRACTOR at least seven (7) days advance notice for the initial mobilization for CONTRACTOR Equipment and CONTRACTOR Personnel.  The subsequent mobilization throughout the duration of CONTRACT will be on twenty-four (24) hours on a "call-out" basis.

I-2.3		CONTRACTOR Contract Administrator

CONTRACTOR hereby appoints, as CONTRACTOR’s Contract Administrator <<To be provided and specified by CONTRACTOR>> or his appointee designated by him in writing.

I-2.4	CLIENT Contract Administrator

CLIENT hereby appoints, as CLIENT Contract Administrator, the Director or his appointee designated by him in writing.

I-2.5	CLIENT Representative(s)

CLIENT hereby designates the Director, Drilling Manager, Drilling Superintendent, Senior Drilling Engineer and Drilling Supervisors as CLIENT Representatives.

I-2.6	Mobilization Site for CONTRACTOR Equipment and Products

The Mobilization Site for CONTRACTOR Equipment shall be the CLIENT’s Warehouse at PTSC Supply Base, Vungtau (now Rach Dua Ward), S.R. Vietnam or any other mutually agreed location.

I-2.7		Demobilization Site for CONTRACTOR Equipment

The Demobilization Site for CONTRACTOR Equipment shall be at CLIENT Warehouse at PTSC Supply Base, Vungtau (now Rach Dua Ward), S.R. Vietnam or any other mutually agreed location.

I-2.8	Mustering Point for CONTRACTOR Personnel

Southern Vietnam Helicopter Company's Helibase in Vung Tau or Ho Chi Minh City (if required by CLIENT) or PTSC Supply Base, Vung Tau (now Rach Dua Ward) for all CONTRACTOR’s Personnel.

I-2.9	CLIENT Warehouse and Base Facility

PTSC Supply Base, Vungtau, S.R. Vietnam.

I-2.10	CONTRACTOR Warehouse and Base Facility

<<To be provided and specified by CONTRACTOR>>

I-2.11	CLIENT Operations Office

15th Floor, Phu My Hung Petroleum Finance Trade Center, 12 Tan Trao Street, Tan My Ward, Ho Chi Minh City, S.R. Vietnam.

I-2.12	CONTRACTOR Operations Office

<<To be provided and specified by CONTRACTOR>>

I-2.13	Commencement Date of Operation

The time and date when CONTRACTOR completed of mobilizing the first CONTRACTOR Equipment or consignment of Products onboard the Drilling Rig at the Mobilization Site or mobilize the first Personnel at the Mustering Point, whichever is earlier.

I-2.14	Commencement of Rates

The applicable rates as defined in EXHIBIT IV hereof shall commence on the Commencement Date of CONTRACT. No payable rates shall be payable throughout mobilization and demobilization of CONTRACTOR Equipment or CONTRACTOR Personnel.

The applicable rates as defined in EXHIBIT IV hereof shall commence upon physical mobilization.

I-2.15	Point of Origin

CONTRACTOR Point of Origin is at Vung Tau  (now Rach Dua Ward) or other location designated by CLIENT for CONTRACTOR's Personnel that are to be furnished under this CONTRACT.

I-2.16	Delivery Schedule

Initial delivery date and the following time CONTRACTOR shall deliver CONTRACTOR Equipment and Products at the designated Mobilization Site on the date stipulated in the notification issued by CLIENT.

All equipment shall be available at the designated Mobilization Site no later than 01 weeks after receipt of CLIENT’s M/R.

All personnel shall be available at the designated Mobilization Site no later than 2 days after receipt of CLIENT’s M/R

CONTRACTOR agrees that CLIENT shall have the right to change or to amend the quantity and delivery date of the CONTRACTOR Equipment.

If other means of transportation is required due to failure of CONTRACTOR to meet committed delivery period, additional cost to be incurred shall be to CONTRACTOR’s account.

I-2.17	Request for Services

A request for Provision of Equipment & Services shall be either via telex, facsimile, telephone or verbal instructions to be followed by telex or facsimile form CLIENT’s Contract Administrator.

I-2.17	Duration of CONTRACT

The duration of the CONTRACT shall be as per ARTICLE 3.

I-2.18	“Call-Out” of CONTRACTOR Personnel

CONTRACTOR shall provide CONTRACTOR Personnel on “Call-out” basis and when directed by CLIENT.  CLIENT shall give to CONTRACTOR at least twenty-four (24) hours for “Call-out” personnel.

I-2.19	“Call-Out” of CONTRACTOR Equipment

CONTRACTOR shall make available CONTRACTOR Equipment at CONTRACTOR Warehouse in Vung Tau, Vietnam for delivery to the Work Site as requested by CLIENT on “call-out” basis.

I-2.20	Completion of WORKS

Completion of WORKS shall be achieved when the operations described in EXHIBIT I-2.0 on the last well designated by CLIENT has been completed and CONTRACTOR has been notified in writing by CLIENT.

I-2.21	Demobilization of CONTRACTOR Equipment and Products

Upon completion of WORKS on the last well designated by CLIENT, CONTRACTOR shall make ready all Equipment and Products for offloading onto the supply vessel(s). CONTRACTOR shall be responsible for the removal of CONTRACTOR's Equipment and Products from the designated Demobilization Site. Rental shall cease when CONTRACTOR Equipment reaches the Demobilization Site.

I-2.22	Release of CONTRACTOR Equipment

CLIENT shall have the right to release any of the CONTRACTOR Equipment for any reason whatsoever within twenty-four (24) hours.  CLIENT shall endeavor to notify CONTRACTOR of its intent to release any of CONTRACTOR Equipment as soon as such decision is known.  However, CLIENT shall continue to pay the applicable rental rates until the CONTRACTOR Equipment has been delivered and reached to the Demobilization Site.  CONTRACTOR shall promptly prepare for demobilization whatever portion of CONTRACTOR Equipment that is to be released and shall assist CLIENT in the loading-out of the said equipment.  CONTRACTOR shall be on hand at the Demobilization Site to take delivery of such equipment.

I-2.23	Re-engagement of CONTRACTOR Equipment

CLIENT shall have the option to re-engage any released CONTRACTOR’s Equipment.

I-2.24	Release of CONTRACTOR Personnel

CLIENT shall give CONTRACTOR at least five (5) days advance notice of release of any designated personnel.  However, subject to terms and conditions of main contact body hereto, CLIENT shall have the right to release without prior notice any of CONTRACTOR’s Personnel, who, in CLIENT’s opinion, are or could be detrimental to the performance of the WORKS or interest of CLIENT.

I-2.25	CONTRACT Closure

CONTRACTOR shall submit to CLIENT a “CONTRACT CLOSURE LETTER” and “CONTRACT CLOSURE CERTIFICATE” as per format shown in APPENDIX X-8 and APPENDIX X-9 respectively.

I-2.26	Health, Safety and Environment Requirement

CONTRACTOR shall provide necessary tools, equipment and written procedure to make sure all hazards listed in APPENDIX VI-5 are addressed.

CONTRACTOR is required to develop the action plan (or written procedure) for each hazard listed in APPENDIX VI-5 (where and when applicable) prior to the commencement of WORKS.

I-2.26	Changes

I-2.26.1	Should CLIENT desire any change to the CONTRACTOR’s Equipment, it shall advise CONTRACTOR of said request.  CONTRACTOR shall, upon request from CLIENT, provide CLIENT seven (7) days a CHANGE PROPOSAL defining the terms and conditions shall include but not limited to price, method of payment, earliest commencement date and any other information deemed necessary.

I-2.26.2	When and if CLIENT approves the CHANGE PROPOSAL, CLIENT will issue to CONTRACTOR a written CHANGE ORDER in duplicate originals in the form shown in APPENDIX IX-2 herein, CONTRACTOR shall sign both duplicate originals of the CHANGE ORDER to indicate its receipt, understanding and acceptance of it. Execution by CLIENT, one duplicate original will be returned to CONTRACTOR.

I-2.26.3	Should the Parties agree to the terms and conditions, including but not limited the Price, and Method of payment, the change shall be made.

I-3.0	SCOPE OF WORKS

The equipment and products listed are proposals only for this operation and do not constitute a commitment by CLIENT to rental or purchase all item. The actual quantities and equipment to be supplied will be detailed in a Call-out instruction issued by CLIENT.

The WORKS is defined and described by, but not necessarily limited to any or all of the following:

I-3.1	Equipment, Personnel and Products

I-3.1.1	CONTRACTOR shall furnish and maintain at the Work Site complete Slickline & MPLT equipment and the necessary CONTRACTOR’s Equipment to conduct efficient Slickline, Mast & MPLT logging related operations at the Work Site(s). CONTRACTOR shall provide CONTRACTOR’s Personnel and technical support, capable of performing the operations as required for the performance of the WORKS but shall not be limited to the following activities:

I-3.1.1.1. Slickline Services for completion:

- Provide suitable tools, optimum bottom-hole assembly (BHA) for Slickline related operations as per CLIENT’s requirements.

- Run Slickline simulations basing on well conditions with selected BHA configuration for each operation.

- Rig up, function and pressure test Slickline Equipment, Tools.

- Calibrate and maintain accuracy of all measuring equipment.

- Conduct Slickline operations such as gauge cutter, clean-out, fishing, swabbing, plug/ prong installation, open/ close sliding side doors, gas-lift valve installation/ replacement etc.

- Onshore works to support completion downhole preparations and tests.

- The Slickline operations are to be carried out 24 hour per day.

- Maintain in good working condition and service all Slickline equipment and tools with 100% back up as required for the operation.

I-3.1.1.2. Slickline & Mast Services for MPLT:

- Propose Slickline & MPLT logging operation programs in detail.

- Propose and provide suitable tools, optimum bottom-hole assembly (BHA) for Slick-line related operations as per CLIENT’s requirements.

- Run slick-line simulations basing on well conditions with selected BHA configuration for each operation.

- Rig up, function and pressure test Slick-line & Mast Equipment, Slick-line Tools & MPLT.

- Calibrate and maintain accuracy of all measuring equipment including depth meter, downhole pressure/temperature recorders, gauges, sensors, thermometers, clocks, flowmeters and gravitometer, etc.

- Conduct Slick-line operations such as closing and opening the sliding side door (SSD), fishing, swabbing, wax/scale cleaning, downhole temperature/pressure surveys, bottomhole sampling, releasing TCP gun assembly, etc.

- Conduct Slick-line operation for MPLT runs to determine downhole flow conditions with sophisticated pressure, temperature, flow, capacitance and density sensors, etc.

- The Slick-line operations are to be carried out 24 hour per day.

- Maintain in good working condition and service all Slick-line, Mast equipment and Logging tools with 100% back up as required for the operation.

- Provide CLIENT with Slick-line, MPLT logging services related consultancy.

- All hardware and software required for the services shall be provided by CONTRACTOR.

I-3.1.2	Upon request by CLIENT, CONTRACTOR shall provide and deliver in good operating condition of Designated “Consignment” and "Call-out" Rental equipment as listed in APPENDIX II-1 or elsewhere in the Published Price List to designated Mobilization Site at any time upon CLIENT’s forty-eight (48) hours or less prior notice to execute the WORKS pursuant to this CONTRACT.

I-3.1.3	CONTRACTOR shall be responsible for the installation, maintenance, inspection, repair or replacement and removal of CONTRACTOR’s Equipment furnished pursuant to this CONTRACT. CONTRACTOR shall at all times maintain adequate facilities, equipment, supplies and spare parts to ensure that CONTRACTOR’s Equipment is in good working condition at all times:

I-3.1.4	CONTRACTOR shall maintain sufficient spares and tools at the Work Site to cope with any failure of CONTRACTOR Equipment.  The list of spares must be submitted to CLIENT Representative prior to spud the first well and shall be updated as spares are utilized.

I-3.2.5	CONTRACTOR shall furnish the inspection reports/ certificates of CONTRACTOR Equipment upon request by CLIENT before delivering to Mobilization Site.

I-3.1.6	CONTRACTOR shall provide adequate number of highly competent personnel as specified in EXHIBIT V for the performance of the WORKS at the Work Site.

I-3.1.7	Upon completion of WORKS, CONTRACTOR Personnel shall make ready all CONTRACTOR Equipment for offloading onto the supply vessel(s).

I-3.1.8	CONTRACTOR shall be responsible for the removal of CONTRACTOR Equipment from the designated Demobilization Site. Rental shall cease when CONTRACTOR Equipment reaches the Demobilization Site.

I-3.1.9	CLIENT shall not, without CONTRACTOR’s consent, authorize any person other than CONTRACTOR Personnel to operate CONTRACTOR Equipment, except in an emergency, in which case, CLIENT may utilize CONTRACTOR Equipment in an appropriate manner.

I-3.1.10	CONTRACTOR shall prepare and submit a copy of monthly equipment report to CLIENT, listing the inventory and status of equipment and tools available at CONTRACTOR's warehouse/disposal complete with but not limited to complete description, serial number, last inspection date, and purchase date.

I-3.1.11	CONTRACTOR shall prepare Equipment Rental Report (ERR) as shown in APPENDIX X-4, listing all equipment on rental to CLIENT.

These sheets, duly signed by the CLIENT Representative shall be presented with the monthly invoice as part of the supporting document.

CLIENT Representative will provide all assistance to ensure these records are accurate and reflect the exact amount of CONTRACTOR Equipment at the Work Site.

I-3.1.12	CONTRACTOR shall maintain an inventory of available stock of Products onboard the rig. CONTRACTOR shall furnish CLIENT's Representative with a clean, legible copy of the Monthly Inventory Report and make the necessary arrangement for shipment of Products to ensure efficient operation.

I-3.1.13	REPORTS:

CONTRACTOR is to furnish a job ticket for each job completed by CONTRACTOR. Tickets must include, as a minimum, the data listed below.

- Location identification giving times and dates of service(s), well name, field, Rig and/or Platform name.

- Names and job titles of all CONTRACTOR personnel involved and dates and time of departure and return from the job. If "on-board" before commencement of the services, state date of arrival and if remaining on board, so indicate.

- Report of any malfunctions of CONTRACTOR's equipment and/or job and reason thereof. If a lengthy report is required, outline the problem on the job ticket and attach two (2) copies of the detailed report.

- Itemized cost breakdown of all costs to be invoiced to the CLIENT.

- Any other items deemed relevant to the project by CONTRACTOR.

Daily report: Daily operation report via email.

Final report including:

- Slickline operation report and tool string diagram

- MPLT operation report and tool string diagrams

- MPLT surveys data.

- MPLT interpretation reports.

- The final report for each well when sent to COMPANY will be made into four (04) color hardcopies and four (04) Flashdrive.

- All reports are to be based on MS Excel and/or MS Word. A hard copy and electronic file are to be provided for each well.

- Final end-of-well reports / recaps (4 copies) will be furnished to CLIENT within fourteen (14) days from the date of Rig release for each well. Well reports are to include conclusions and recommendations.

I-3.2	Services and Technical Support

I-3.2.1	CONTRACTOR shall use CONTRACTOR Equipment and CONTRACTOR Personnel to perform the WORKS in accordance with CLIENT requirements.

I-3.2.2	CONTRACTOR shall also perform any other related services normally provided by CONTRACTOR within CONTRACTOR's capabilities, which CLIENT may reasonably request.

I-3.2.3		CLIENT will be responsible for transportation of CONTRACTOR Equipment from designated Mobilization Site to Work Site and vice-versa.

I-4.0	WORK ORDER REQUEST

Work Order Request applicable for requesting additional Equipment/ Materials/ Services to be furnished pursuant to this CONTRACT but the rates have not been specified in EXHIBIT IV – CONTRACT PRICE AND PRICE LIST.

Any request for Equipment/ Materials/ Services under Work Order Request category shall within the scope of this CONTRACT and shall be made in writing using WORK ORDER REQUEST format shown in APPENDIX X-6.

The Work Order Request shall state the scope of each "Equipment/ Materials/ Services", Work Site, commencement date, applicable rates, reimbursable cost to CONTRACTOR by CLIENT and other details with respect to the Work Order Request and should there be a conflict, the CONTRACT shall prevail.

I-5.0	CONTRACTOR Work Performance Evaluation

CLIENT shall continuously evaluate the work performance of CONTRACTOR pursuant to the requirement of the CONTRACT throughout the term of the CONTRACT.

I-6.0	Confidentiality of Information

All documents, data and other information which have not been made public and which come into CONTRACTOR’s possession or to his attention in the course of rendition of the SERVICES shall be CLIENT’s sole property and shall not be used in whole or in part by CONTRACTOR other than as necessary hereunder or released or disclosed in whole or in part by CONTRACTOR to anyone not employed by CLIENT without CLIENT’s written consent (except where required by court order or subpoena, copies of which shall be timely furnished to CLIENT prior to release or disclosure). Without CLIENT’s prior written consent, neither CONTRACTOR nor any person acting for or on behalf of CONTRACTOR will confirm or deny any statement to any third parties regarding any information provided to CONTRACTOR hereunder or disclose to any third party that such information has been provided to CONTRACTOR. All reports, documents, data, presentations and other material prepared by CONTRACTOR pursuant to this Agreement shall be CLIENT’s sole property in which CONTRACTOR shall have no proprietary rights (including, but not limited to, copyright). Upon termination of this Agreement, CONTRACTOR will promptly deliver to CLIENT all documents, data and other information and all copies of same, and any analyses, compilations, studies or other data prepared hereunder in CONTRACTOR’s possession or control. This confidentiality obligation, which shall be strictly construed, shall survive the termination of this Agreement. CONTRACTOR will require all its employees and Sub-CONTRACTORs to agree to and abide by terms identical to this Item.

I.7.0	Training of CLIENT personnel

CLIENT seeks opportunity for the training of its personnel in the well testing services and related operations. Accessibility to CONTRACTOR’s training facilities within or outside Vietnam will be a consideration in the bid evaluation exercise. All expenses incurred by CLIENT personnel would be on account of the CLIENT.

I-8.0	Technology Transfer Incentive (TTI)

CONTRACTOR is encouraged to submit to CLIENT a TECHNOLOGY TRANSFER INCENTIVE (TTI) plan, it also will be a consideration in the bid evaluation exercise. This shall include but not limited to Manufacturing site visits, technology training for CLIENT’s personnel, technical workshops.