EXHIBIT VI

HEALTH, SAFETY AND ENVIONMENT REQUIREMENT

| TABLE OF CONTENTS |  |
| --- | --- |
| SECTION | TITLE |
| VI-1.0 | SAFETY POLICY & PROGRAM |
| VI-2.0 | ACCIDENT REPORTING & INVESTIGATION |
| VI-3.0 | ALCOHOL/DRUG POLICY |
| VI-4.0 | PERMIT TO WORK SYSTEM |
| VI-5.0 | SAFETY TRAINING & SUPERVISION |
| VI-6.0 | PERSONNEL PROTECTIVE EQUIPMENT |
| VI-7.0 | SAFETY DRILL |
| VI-8.0 | SAFETY MEETING |
| VI-9.0 | MEDICAL WELFARE |
| VI-10.0 | TOOL & EQUIPMENT |
| VI-11.0 | HOUSEKEEPING |
| VI-12.0 | ENVIRONMENTAL PROTECTION |
| VI-13.0 | LAND TRANSPORTATION |
| VI-14.0 | OFFSHORE SAFETY PASSPORT |
| VI-15.0 | TYPICAL HAZARDS |
| APPENDIX |  |
| VI-1 | APPROVED PERSONNEL PROTECTIVE EQUIPMENT |
| VI-2 | TRAINING/CERTIFICATION REQUIREMENTS |
| VI-3 | APPROVED SAFETY TRAINING INSTITUTIONS |
| VI-4 | MONTHLY SAFETY STATISTICS REPORT |
| VI-5 | LIST OF TYPICAL HAZARDS |

EXHIBIT VI

HEALTH, SAFETY AND ENVIRONMENT REQUIREMENT

This EXHIBIT provided a reference to requirements HSE Management System applicable to the CONTRACT.  The list is by no means definitive and the CLIENT Safety Manual together with other policies, procedures and special instructions, shall also apply to this CONTRACT.

When this EXHIBIT and its APPENDICES refers to the CONTRACTOR and the CONTRACTOR's Personnel, this shall also be taken to refer to Sub-contractors of the CONTRACTOR and the Sub-contractor's Personnel.

CONTRACTOR shall prepare the COVID-19 Preparedness & Response Plan and must cover all issues and cost related to the Covid-19 throughout the duration of contract. This Plan has been established for the Bids and contract in accordance with National Steering Committee for Prevention and Control of Covid-19 and CLIENT’s Guideline. Prior to performing Bids or contract, Contractor send The COVID19 Preparedness & Response Plan to CLIENT for reviewing and approving. Moreover, in the event that there is mandatory quarantine requirement from the government of Vietnam during the performance period of the contract, no CONTRACTOR’s personnel and/or equipment rates during such mandatory quarantine period shall be applied and CONTRACTOR shall pay for meal & accommodation, appointed by the authority, & testing in Vietnam. If any of CONTRACTOR’s personnel is concluded infected to the aforesaid virus and/or disease, CONTRACTOR shall immediately send replacement and bear all the related cost including but not be limited to CONTRACTOR’s personnel & equipment time waiting for such personnel substitute, mob/demob cost, meal & accommodation and testing and medical treatment cost of the infected personnel.

VI-1.0    	HSE POLICY & PROGRAM

CONTRACTOR shall have in place, prior to the commencement of this CONTRACT, a written policy on Safety, Health and Environment. This policy shall be of a standard comparable to CLIENT’s policy and shall be actively supported and endorsed by CONTRACTOR's Management.  A copy to be submitted to CLIENT prior to the commencement of the operation.

In addition, CONTRACTOR shall also have safety targets and objectives, paying particular attention to accident prevention, written action plans clearly indicating how safety objectives shall be achieved and a system to appraise risk and problem areas to ensure that the overall safety program is being carried out and complied with.

CONTRACTOR shall submit the Monthly Safety Statistics Report as per format shown in APPENDIX VI-2.

VI-2.0 	ACCIDENT REPORTING & INVESTIGATION

VI-2.1	Accident is defined as any unintentional or unplanned event or condition which has or could have resulted in injury to a person, loss or damage to equipment, plant or property, or pollute to environment.

VI-2.2 	CLIENT requires that all accidents, no matter how trivial, must be reported to the CLIENT’s Representative(s). CONTRACTOR shall ensure that its employees are aware of this mandatory requirement.

CONTRACTOR shall be responsible to investigate, in a professional manner, all accidents that occur during the performance of the WORKS.  The investigation report shall be made available to CLIENT when requested. CONTRACTOR shall also be responsible for assisting CLIENT in accident investigations, if so required.

VI-3.0    	ALCOHOL/DRUG POLICY

VI-3.1 	CONTRACTOR ensures that its employees, agents and Sub-contractors shall not perform any WORKS for CLIENT while under the influence of alcohol or any controlled substance. CONTRACTOR, its employees, agents, and Sub-contractors shall not misuse legitimate drugs or possess, use, distribute, or sell illicit or un-prescribed controlled substances or drugs on CLIENT business or premises.  CONTRACTOR shall adopt and enforce WORKS rules and policies in order to assure compliance with these obligations.

VI-3.2 	CLIENT reserves the right to conduct alcohol and/or drug tests on the CONTRACTOR employees, agents, or Sub-contractors while on premises owned or controlled by CLIENT's where reasonable cause exists.

CLIENT also reserves the right to conduct searches for possession of drugs and/or alcohol on the person, vehicles, and other property of CONTRACTOR, its employees, agents or Sub-contractors while on premises owned or controlled by CLIENT. Any person who refuses to cooperate with any such search shall be removed from the premises and not permitted to return.

VI-3.3	CONTRACTOR shall require its employees, agents and Sub-contractor to submit to medical evaluation or alcohol or drug testing where cause exists to suspect alcohol or drug use.

VI-3.4 	CONTRACTOR warrants that any employee, agent or Sub-contractor who either: (1) refuses to participate in medical evaluation or alcohol or drug tests, or (2) tests positive for alcohol or a controlled substance, shall be removed from the premises and not be permitted to perform any WORKS for CLIENT.

VI-4.0   	PERMIT-TO-WORK SYSTEM

All WORKS carried out by CONTRACTOR's Personnel must be in accordance with the CONTRACTOR “Permit-To-Work System".

VI-4.1   	Hot-Work

All Hot-Work must be carried out in strict compliance accordance with the CONTRACTOR's "Permit-To-Work System" and attention must be given to the following:

VI-4.1.1	Only trained, competent personnel shall be engaged for any Hot-Work.

VI-4.1.2 	The personnel carrying out the WORKS must have a copy of the authorized Hot-Work permit displayed at the Work Site(s) or Work Area.  This permit shall clearly state the nature of the WORKS to be carried out, safety precautions to be taken regarding fire equipment, personal protective equipment, etc., and necessary tests for the presence of combustible gases.

VI-4.1.3 	The CONTRACTOR, in accordance with the "Permit-To-Work System" shall dedicate a trained fire-watcher to the personnel performing the WORKS. The duties of the fire-watcher are as follows (if applicable):

- To ensure that the Work Site(s) or Work Area and adjacent areas are maintained in a safe condition (e.g., sparks are not falling onto unprotected area).

- To warn the workers of any hazard developing in the Work Site(s) or Work Area.

- To isolate the equipment in the event of an emergency.

- To quickly extinguish any smouldering material which may develop into a fire.

- To be able to establish contact with the Control Room if an emergency occurs.

- To ensure that a minimum of one dry chemical fire extinguisher, one pressurized fire hose, and a fire blanket are available at the Work Site(s) or Work Area and be capable of using this equipment if required to do so.

- To ensure that the Work Site(s) or Work Area is clear of all flammable and combustible material prior to commencement of WORKS.

VI-5.0 	SAFETY TRAINING & SUPERVISION

Prior to the commencement of the WORKS, CONTRACTOR shall at its own expense ensure that its personnel have been given the necessary basic safety, fire-fighting, sea-survival, and job related training required by law and CLIENT as outlined in APPENDIX VI-2.  Such training shall be carried out at training establishments approved by CLIENT as listed in APPENDIX VI-3. Certification of training shall be provided to CLIENT prior to the commencement of WORKS.

Upon request from CONTRACTOR, CLIENT may approve training provided by training establishments other than those specified in APPENDIX VI-3 provided CONTRACTOR furnishes CLIENT a copy of the course/program syllabus and contacts for the establishment.

VI-6.0 	PERSONAL PROTECTIVE EQUIPMENT

VI-6.1 	CONTRACTOR shall at its own expense be responsible for providing its personnel with CLIENT approved Personal Protective Equipment suitable for the task being carried out.

VI-6. 2	CONTRACTOR shall ensure that its personnel and/or other personnel assigned by the CONTRACTOR to the performance of the WORKS wear the following minimum Personal Protective Equipment when engaged in WORKS or when in an on-site area where such equipment is required:

- Coveralls,

- Safety footwear with steel toe cap and steel mid sole,

- Safety helmet, and

- Safety glasses.

VI-6.3	Other Personal Protective Equipment that is required, depending on the nature of the job to be carried out and the likely hazards encountered in certain WORKS environments, including:

- Ear muff/plugs,

- Gloves,

- Face shield/Visor,

- Respiratory protection, and

- Fall protection.

VI-6.4	CONTRACTOR must ensure that the standard Personal Protection Equipment such as gloves, apron and respiratory protection equipment issued to its personnel complies with the recommendations contained in the manufacturer's "Material Safety Data Sheet" for the product.

NOTE: 	In some circumstances this may require the wearing of self-contained or air-line breathing apparatus and the CONTRACTOR must ensure that these employees are thoroughly familiar with all types of respiratory protection.

VI-6.5	A list of CLIENT Approved Personal Protective Equipment is contained in APPENDIX VI-1.

VI-7.0 	SAFETY DRILL

VI-7.1 	The CONTRACTOR shall ensure that its personnel are thoroughly familiar with all site alarms, their muster station and, where applicable, the location of their lifeboat.

VI-7.2 	CONTRACTOR's Personnel should rapidly acquire a thorough knowledge of site escape routes including alternative routes if the primary route happens to be blocked.

VI-7.3 	The employees, servants or agents of the CONTRACTOR when working offshore or onshore shall participate in appropriate emergency drills and emergency response training (e.g., fire, muster and boat drills, instructions in survival, lifesaving, and firefighting).

VI-8.0	SAFETY MEETINGS

The CONTRACTOR’s Personnel shall, from time to time, be requested to attend CLIENT safety meetings.  Attendance at these meetings is mandatory and does not relieve the CONTRACTOR of the obligation to hold its own safety meetings as outlined in CONTRACTOR's safety policy.

VI-9.0 	MEDICAL WELFARE

VI-9.1 	CONTRACTOR shall ensure that all its employees and Subcontractor's employees engaged in the WORKS are medically fit and healthy.  Any medical disabilities, including such disabilities which CONTRACTOR may consider will not adversely influence the employee's ability to perform his role in the WORKS, shall be reported to CLIENT prior to the start of the WORKS. CONTRACTOR, if requested by CLIENT, shall provide medical certificates for CONTRACTOR and Sub-contractor personnel.

VI-9.2 	CONTRACTOR shall, at no cost to CLIENT, be responsible for the medical welfare of its own and Sub-contractor's employees and shall take care of arrangements for medical attendance, treatment or hospitalization if and when necessary and will arrange suitable insurance coverage for such contingencies. In cases of emergency, CLIENT may make or provide for, the necessary emergency arrangements, the costs of which shall be reimbursed to CLIENT by CONTRACTOR.

VI-10.0 	TOOLS & EQUIPMENT

Equipment and tools shall only be operated by competent personnel.

CONTRACTOR shall ensure that all CONTRACTOR's machinery, equipment, facilities, and other items associated with or utilized in the WORKS are maintained in a safe, sound and proper condition, and comply with laws, regulations, and CLIENT requirement.

CONTRACTOR shall ensure that all tools and equipment and temporary facilities and other items used in the WORKS, whether purchased, rented or otherwise provided by CONTRACTOR are in a safe, sound and good condition and are capable of performing the function for which they are intended.

If any tool or item of equipment provided by CONTRACTOR is unsafe or incapable of doing the WORKS, CONTRACTOR shall repair and/or replace such defective tools and equipment used in the WORKS at CONTRACTOR's own expense.

VI-10.1	DIESEL ENGINE DRIVEN PUMPS, COMPRESSORS, WELDING SET, ETC.

VI-10.1.1 	Diesel engine driven pumps, compressors, welding sets, and any other diesel engine driven equipment must be located in a safe area.

VI-10.1.2 	Any such equipment must be well maintained and in good operational order and prior to use should be inspected by CLIENT Representative.

VI-10.1.3 	If the equipment is to be used offshore, it shall be inspected prior to shipment to the offshore site.

VI-10.1.4 	If such equipment is to be used in a hazardous area classified as a Zone 1 or 2 area, the equipment must comply fully with EEMUA 107 requirements (Recommendations For The Protection Of Diesel Engines Operating in Hazardous Areas).  The said requirements were formerly contained in an Oil Companies Materials Association (OCMA) publication.

VI-10.2	ELECTRICALLY POWERED PORTABLE TOOLS AND Equipment

VI-10.2.1	Only trained and competent personnel shall use portable electrically powered tools and equipment.

VI-10.2.2	Electrically powered portable tools and equipment shall be 110 volts centre-tapped or 240 volt tools subject to the following conditions:

- they shall be double insulated, and

- the power source shall be equipped with an earth leakage circuit breaker/ ground fault interrupter (ELCB/GFI) with a trip sensitivity not exceeding ten milli-amps (10mA).

VI-10.2.3	A written procedure for checking and maintaining portable electrically powered hand tools and equipment shall be in place.  Any defective electrical tool and equipment shall be immediately prohibited from further use until it has been satisfactorily repaired. Defective items which are no longer serviceable shall be removed from storage or use and scrapped. CONTRACTOR shall maintain accurate records regarding the maintenance and disposal of such equipment.  The CONTRACTOR shall appoint a person(s) to be responsible for such maintenance.

VI-10.3 	HANDTOOLS

VI-10.3.1 	CONTRACTOR shall ensure that all tools supplied are in good condition and fit for their intended use.

VI-10.3.2	Damaged tools which are unfit for use must be removed from service immediately and if they cannot be repaired, they must be scrapped.

VI-10.3.3	CONTRACTOR's employees must be fully instructed regarding the use of the correct tool for a particular job, (e.g. The use of a cheater bar or piece of pipe to increase the length of a pipe wrench handle is totally unacceptable.  A larger pipe wrench must be used).

VI-11.0	HOUSEKEEPING

VI-11.1	CONTRACTOR shall ensure that its personnel keep and maintain good housekeeping practices at the Work Site to eliminate all hazards or control hazardous conditions in order to avoid injury to workers throughout the duration of the WORKS.  CONTRACTOR shall know what the hazards are and how to guard against the hazards, the kind of which shall include unsafe acts and unsafe conditions.

VI-11.2  	In order to reduce the risk of fire, waste materials and garbage shall not be allowed to accumulate and as a minimum, must be disposed of on a daily basis and in an appropriate manner.

VI-11.3 	In order to minimize site hazards (such as trips, slips, falls, etc.), access ways must be kept clear of electrical cables, wires, metal pipes, scaffold boards and other materials and equipment.

VI-11.4	All gaps such as that caused by the removal of gratings must be adequately roped off and the grating stored so that it does not become an "obstruction and trip hazard".

VI-12.0	ENVIRONMENTAL PROTECTION

VI-12.1	CONTRACTOR shall pay due regard to the environment by acting to protect air, water, animal and plant life from adverse effects of CONTRACTOR's activities, and to minimize any adverse effects which may arise from such operations in accordance with government and CLIENT environmental policies.

VI-12.2	CONTRACTOR shall adhere to existing national statutory regulations concerning discharges resulting from the performance of the WORKS.

VI-12.3	CONTRACTOR and its Sub-contractors shall not, under any circumstances dump, throw or dispose of any refuse, oily wastes, toxic substance, debris or garbage into the sea. CONTRACTOR shall provide containers in which all refuse is to be placed and shall dispose of such refuse in accordance with existing laws and regulations.

VI-12.4	CONTRACTOR shall ensure that its employees and its Subcontractors, and their employees are fully aware of the above and CONTRACTOR shall enforce such regulations to the satisfaction of CLIENT.

VI-13.0	LAND TRANSPORTATION

VI-13.1	CONTRACTOR's Personnel travelling to and from onshore work site(s) shall use proper and safe means of transport.

VI-13.2 	Such transport shall comply with the Vietnamese traffic rules and regulations.

VI-13.3	Transportation of the CONTRACTOR's personnel in open trucks is prohibited unless the truck is fitted with seats and certified to carry passengers.  All speed limits must be adhered to.

VI-14.0	TYPICAL HAZARDS

VI-14.1	CONTRACTOR is required to provide the tools and equipment as well as written procedure to ensure all hazards listed in APPENDIX VI-3 are addressed.

VI-14.2	CONTRACTOR is required to develop an action plan (or written procedure) for each mentioned hazard (where and as and when applicable) prior to commencing the WORKS.

END OF EXHIBIT

APPENDIX VI-1

PERSONAL PROTECTIVE EQUIPMENT

1.0	SAFETY SPECTACLES

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| KING OR EQUIVALENT |  | ANSI Z78.1 |

2.0     	SAFETY BOOTS

  - FOR GENERAL USE

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| KRUSHERS OR EQUIVALENT |  | AS/NZS 2210.3: 2000 ASTM F2413-05 EN 345-1 |

2.2  FOR CHEMICAL USE

RUBBER BOOT - VIETNAM

3.0     COVERALL

FRABRIC 100% COTTON

4.0     HAND PROTECTION

4.1  IMPACT GLOVES

4.2  WOOL GLOVES WITH PLASTIC DOT

4.3  WELDER GLOVES

COW-HIDE, COLOR REUST, DELUXE, COTTON LINING

5.0     HEARING PROTECTION

  - EAR MUFFS MOUNTED ON SAFETY HELMETS

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| PROTECTOR OR EQUIVALENT | CLASS 5 | ANSI S3.19-1974/CSA Z94.2 |

5.2 EAR PLUGS

3M	                  EAR PLUGS 1110

OR EQUIVALENT

6.0     HEAD PROTECTION

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| BULLARD OR EQUIVALENT | CLASS E, TYPE I | ANSI/ISEA Z89.1-2009 |

7.0     EYE/FACE PROTECTION

7.1  CHEMICAL SPLASH GOGGLES

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| CONDOR OR EQUIVALENT | 1VT70 |  |

  - FACE SHIELD

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| MSA OR EQUIVALENT | 488126 | ANSI Z87 |

7.3  WELDERS GOGGLES

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| GRAINGER OR EQUIVALENT | PN 1UYF9 |  |

8.0     OTHERS

8.1  SAFETY HARNESS/LIFEBELT

| BRAND | MODEL NO. | STANDARD |
| --- | --- | --- |
| SALA OR EQUIVALENT | 1107802 | ANSI Z359.1 ANSI Z359.3 ANZI Z359.4 |

8.2  DUST MASK (DISPENSABLE)

VIETNAM

8.3  RAIN WEAR

VIETNAM

8.4  WELDERS APRON

ANSELL OR EQUIVALENT

8.5  CHEMICAL - HANDLING APRON

ANSELL OR EQUIVALENT

  - WORK VEST

STEARNS OR EQUIVALENT           I223                     US COAT GUARD

(TYPE V)                                                                            APPROVED

APPENDIX VI-2

TRAINING/CERTIFICATION REQUIREMENTS

| NO. | COURSE TITLES | ATTENDANCE GROUP | MIN. COURSE | VALIDITY |
| --- | --- | --- | --- | --- |
|  |  |  | DURATION |  |
| 1. | Supervisor Safety | All supervisory personnel supervising minimum of 8 employees | 3 Days | - |
| 2. | Rigging and Slinging Safety | All personnel who are assigned rigging/slinging responsibility. | 3 Days | - |
| 3. | Scaffolding | a) All supervisory personnel who are required to supervise erecting and dismantling of scaffolding and inspect scaffolds | 10 Days | - |
|  |  | b) All personnel erecting or dismantling scaffolding. | 10 Days | - |
| 4. | Basic Safety, Sea Survival and Fire & Fighting Courses | All personnel who spend 5 or more nights offshore at any one Visit or exceeding more than 20 days cumulative per year. | 5 Days | 3 Years |
| 5. | Crane Operator's Course | All personnel who are required to operate cranes. | 3 Days | 3 Years |
| 6. | Forklift Driver's Course | All personnel who are required to operate forklift. | 1 Day | 3 Years |
| 7 | Gas Tester | The personnel shall appoint to carry out the job |  |  |
| 8 | Medic | The personnel shall appoint to carry out the job |  |  |
| 9 | Electrician | The personnel shall appoint to carry out the job |  |  |
| 10 | Welders | The personnel shall appoint to carry out the job |  |  |
| 11 | Radiation Protection Officer and Radiation Protection Supervisor | The personnel shall appoint to carry out the job |  |  |
| 12 | Chemical Handling | The personnel shall appoint to carry out the job |  |  |
| 13 | Anchor Handling | The personnel shall appoint to carry out the job |  |  |
| 14 | Authority to Inspect | The personnel shall appoint to carry out the job |  |  |

APPENDIX VI-3

APPROVED SAFETY TRAINING INSTITUTIONS

Name of Institution                    	Courses

1.	Sri Bima Maritime Training      			o	Sea Survival & Fire

Centre, Miri, Sarawak             				Training.

2.	Terengganu Safety Training Centre		o	Safety & Sea Survival

(TSTC), Teluk Kalong					Fire Training        Kemaman,Terengganu		      	 		HUET

3.	Kerteh Fire Services  				o	Fire Training

East Coast Regional Office

PETRONAS Kerteh, Terengganu

4.	Robert Gordon Institute				o	Sea Survival & Fire

of Technology (RGIT)           				Training.

Aberdeen, Scotland             				HUET

5.	Petroleum Training Assoc. 			o	Sea Survival &

North Sea (PETANS)        					Safety

Lowestoft, England       					HUET

6.	Maritime Training Centre 				o	Sea Survival & Safety

(MTC) Vlissingen, Holland				o	HUET

7.	Rotterdam International				o	Fire Training

Safety Centre (RISC), Holland

8.	Industrial Foundation				o	Sea Survival, Fire

For Accident Prevention				o	Training & Safety

Woodside Offshore Petroleum			o	HUET

Perth, Australia

9.	Alert Disaster Control (Asia)			o	Safety and Sea Singapore							o	Survival. Fire Training.

10.	PetroVietnam Manpower Training College 	o	Safety and Sea Survival

HCMC, Vietnam 				o	Fire Training

o	HUET

o	HUET

11.	PVD Training					o	Safety and Sea Survival

HCMC, Vietnam 				o	Fire Training.

NOTE:		In certain cases it may not be practical to send personnel to some of the above training establishments due to the logistical problem.  However, a number of the above establishments can be contracted to train CONTRACTOR Personnel at the Work Site(s).

LIST OF APPROVED MEDICAL EXAMINERS

1.	SOS International Centre

65 Nguyen Du, Sai Gon Ward, HCMC, Vietnam.

- Columbia Saigon Clinic

08 Alexandre de Rhodes, Sai Gon Ward, HCMC, Vietnam

- VSP Medical Centre

Vietsovpetro Clinic

5 Area, Tam Thang Ward, HCMC

- Victoria Health Care

79 Dien Bien Phu Street,, Sai Gon Ward, HCMC, Vietnam

APPENDIX VI-4

MONTHLY SAFETY STATISTICS REPORT

TO: 	NAM CON SON PETROLEUM EXPLORATION PRODUCTION BRANCH  - PETROVIETNAM EXPLORATION PRODUCTION CORPORATION LIMITED

15th Floor, Victory Tower, 12 Tan Trao Street, District 7, Ho Chi Minh City, S.R. Vietnam

ATTN : 	DRILING MANAGER

MONTH 	: ______________   	CONTRACT NO. __________________

CONTRACTOR  : ______________  	WORK AREA   : __________________

| No. | Description | Units | Area |
| --- | --- | --- | --- |
| 1 | Total Man-hours worked (including CONTRACTOR man-hours) | hrs |  |
| 2 | Days without LTI | day |  |
| Lost Time Injury |  |  |  |
| 3 | Fatality (FT) | case |  |
| 4 | Permanent Total/Partial Disability (PTPD) | case |  |
| 5 | Lost Workday Case (LWDC) | case |  |
| Non-Lost Time Injury |  |  |  |
| 6 | Restricted Workday Case (RWDC) | case |  |
| 7 | Medical Treatment Case (MTC) | case |  |
| 8 | First Aid Case (FAC) | case |  |
| HSE indicators |  |  |  |
| 9 | Lost Time Injury Frequency (LTIF) =(total LTI cases) x 1,000,000 / (total man-hours) | case per 1,000,000 man-hrs worked |  |
| 10 | Total Recordable Injury Frequency (TRIF) =(Total Recordable Injuries) x 1,000,000 / (total man-hrs) | TRI per 1,000,000 man-hrs worked |  |
| 11 | Days lost for LWDC | days |  |
| 12 | Severity of LWDC =(Total lost days for LWDC) / (total LWDC) | days/case |  |
| Non-Injurious incident |  |  |  |
| 13 | Fire / Explosion incident | case |  |
| 14 | Property Damage incident (PD) | case |  |
| 15 | Hydrocarbon Release Incident | case |  |
| 16 | Chemical spill incident | case |  |
| 17 | Oil Spill to environment | case |  |
| 18 | Occupational Illness (OI) | case |  |
| 19 | Other Non-Injurious incident (See Definition below) | case |  |
| HSE performance efforts |  |  |  |
| 20 | Near-Miss Report | ea |  |
| 21 | Safety observation reports | ea |  |
| 22 | Emergency Drills/Exercises | ea |  |
| 23 | Permit-To-Work | ea |  |
| 24 | Audits and inspections | ea |  |
| 25 | Safety meetings (including tool-box talks) | ea |  |
| 26 | Job risk assessments (JSA, STING, TRA…) | ea |  |
| Environmental report |  |  |  |
| 27 | Hazardous waste | Kg |  |
| 28 | Non-Hazardous waste | Kg |  |
| 29 | Produced water discharge | m3 |  |
| Others |  |  |  |
| 30 | Cost of incidents burdened by PVEP-NCS | USD |  |
| 31 | Shutdown (incident) | case |  |
| 32 | Downtime due to HSE incident | hrs |  |

SIGNATURE  	: _______________   DATE        : _______________

NAME    	: _______________   DESIGNATION : _______________

DEFINITION OF ACCIDENT TYPE

1.	LOST TIME INJURY (LTI)

Any work related injury or illness which renders the injured person unable to perform his normal duties, on any day immediately following the day of the accident.  It can be divided into four categories:

i)	Fatality

Death due to work related injury or illness regardless of the time between injury or illness and death.

ii)	Permanent Total Disability

A work related injury which incapacitates a person permanently and results in termination of employment.

iii)	Permanent Partial Disability

A work related injury which results in the complete loss or permanent loss of use of any member or part of the body or any permanent impairment of functions of parts of the body, regardless of any pre-existing disability of the injured member or impaired body function.

iv)	Lost Workday Case

A work related injury or illness other than a Permanent Partial Disability which renders the injured person temporarily unable to perform his normal duties, on any day immediately following the day of the accident.

2.		NON LOST TIME INJURY (NLTI)

Any work related injury or illness other Lost Time Injury.  It can further divide into three categories:

i)	Restricted Work Case

A work related injury or illness which requires the person to be treated by doctor and is declared fit to return to do part of his normal work on restricted or light duties on the day immediately after following the accident/injury. (N/B. Restricted activity/light duties must be within the injured person's normal scope of work.)

ii)	Medical Treatment Case

A work related injury or illness that involves neither lost workdays nor restricted workdays but which requires the injured to be treated by or under the specific orders of a physician or could be considered as being in the province of a physician.

iii)	First Aid Injury

A work related injury or illness which requires minor treatment or subsequent observation of minor scratches, cuts, burns, splinters and so forth which do not ordinarily require medical care even though such treatment is provided by a physician or registered professional personnel, and the injured is able to resume his normal job function immediately after treatment.

3.	FIRE/EXPLOSION

Any event involving fire/ explosion.

4.	PROPERTY LOSS/ DAMAGE

Any accident which involves property damage/loss other than a fire or explosion.

5.	OIL SPILL

Any accident involving spillage or release of oil and/or chemicals which cause pollution of air, water or ground.

6.	NEAR MISS

Any event or condition which whilst not resulting in injury to person or loss or damage to equipment, plant or property, or pollution to the environment is recognized as having the potential to cause injury and/or property damage.

7.	STOP CARD (Safety Observation Card)

Any Safe/Unsafe Act/Unsafe Condition that are reported on STOP Cards. Count each card as one effort

8.	OTHERS

Any Incident/Accident is not above-mentioned

LTI FREQUENCY RATE = 	No. of LTA * 1,000,000

Man-hours Worked

LTI SEVERITY RATE  =	 No. of Days Lost * 1,000,000

Man-hours Worked

APPENDIX VI-4

TYPICAL HSE HAZARD

| HEALTH | SAFETY | ENVIRONMENT |
| --- | --- | --- |
| ## Hazardous Materials | ## Fire and Explosion | ## Airborne Emissions |
| - Sulphuric Acid | - Flammability | - Vents (CO2, CH4) |
| - Caustic Soda | - Expansion | - Flares |
| - TENORM | - BLEVE | - Fugitives |
| (Technologically Enhanced Naturally Occurring | (Boiling Liquid Expanding Vapour Explosion) | - General Exhaust |
| Radioactive Material) | - VCE (Vapour Cloud Explosion) | ## Underground Equipment Failure |
| - Hydrogen Sulphide |  | - Corrosion soil |
| - Chlorine | ## Flammable Properties | - Erosion contamination |
| - Nitrogen | - Crude Oil |  |
| - Solvent Fumes | - Hydrogen Gas | ## Jetty/ Ship Operation |
| - Mercury | - Hydrogen Sulphide | - Hoses/ Boom |
|  | - Hydrogen | - SBMs (Single Buoy Mooring) |
| ## Asphyxiation | - Condensate | - Ballast Disposal |
| - Nitrogen |  | - Spillages |
| - Oxygen Deficiency | ## Ignition Sources |  |
|  | - Electrical | ## Surface Water Run-offs |
| ## Radiological | - Stray Currents | - Contamination of waterways |
| - Instrumentation | - Static |  |
| - Inspection | - Lightning | ## Process Effluents |
| - TENORM (Technologically Enhanced | - Pyrophics - Smoking | - Recovered Oils/ Ballast - Produced water – barium, zinc, |
| Naturally Occurring Radioactive Material) | - Welding - Grinding/ Cutting | ## trace of radioactive materials - Condensate water from |
|  |  | Slug Catchers |
| ## Lightning | ## Fire Types | - Surge Vessel Liquids |
| - Lightning Strikes | - Jet, Pool and Flash Fires | - From Drains |
|  | - Lagging Fires | - Triethylene Glycol |
| ## Burns | - Smoke | - Biocides (glutaraldehyde) |
| - Hot & Cold Material |  | - Magnesium Hydroxide |
| Equipment | ## Water |  |
|  | - Low Points |  |
|  | - Draining |  |
|  | - Hydrates |  |
|  | - Emulsions/ Tank Layering |  |
|  | - Dead Legs |  |
|  | ## Air |  |
|  | - Start-Up/ Shut-Down |  |
|  | - Flare Systems |  |